Angle of Arrival (AoA) and Angle of Departure (AoD)

Bluetooth® Specification Change Request

- Date [Publish Date]2016-Jun-21[Publish Date]

- Revision [Status]R08[Status]

- Group Prepared By Core Specification Working Group (CSWG)

- Feedback Email core-main@bluetooth.org

- Impacted Specifications BLUETOOTH SPECIFICATION/Version 4.2

Abstract: This document specifies the changes to the Architecture, Acronyms & Abbreviations, HCI, Physical Layer, Link Layer, Message Sequence Charts, Direct Test Mode sections of the Core specification as a part of incorporating the Angle of Arrival (AoA) and Angle of Departure (AoD) features, for both Connectionless and Connection-Oriented modes of operations, to the v4.2 Core specification.

# Revision History

| Revision History |  | Date |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| R00 |  | 2015-12-15 |  | Initial Draft |  |
| R01 |  | 2016-01-20 |  | Address comments from Angel and Clive. |  |
| R02 |  | 2016-01-26 |  | Address further comments from Clive. |  |
| R03 |  | 2016-01-27 |  | Contributions and further comments from Clive. |  |
| R04 |  | 2016-01-29 |  | Made changes corresponding to the comments resolved on the conference call on 28 Jan. Add MSCs. |  |
| R05 |  | 2016-01-30 |  | Address a few comments from Joel. Describe how AoD calculations can be done. |  |
| R06 |  | 2016-02-10 |  | Comment resolutions from the Deerfield Beach F2F. |  |
| R07 |  | 2016-04-29 |  | Address comments from Harish. |  |
| R08 | 2016-06-21 | Address comments from: - BARB (Rasmus (two sets of comments), Per) - Core (Clive, Florian) - Technical Editors (Chris, Catherine) |
| R09 | 2016-06-28 | Change the Version number for all new HCI commands from Shanghai to Madrid. |

# Contributors to Connectionless AoD

| Name | Company |
| --- | --- |
| Jason Hillyard | ARM |
| Stephen Coe | BlackBerry |
| Angel Polo | Broadcom |
| Raja Banerjea | CSR |
| Mayank Batra | CSR |
| Ian Blair | CSR |
| Robin Heydon | CSR |
| Sabita Nahata | CSR |
| Neil Stewart | CSR |
| Jonathan Tanner | CSR |
| Harish Balasubramaniam | Intel |
| Seung R. Yang | LGE |
| Jiao Xianjun | Nokia |
| Zhang Xin | Nokia |
| Juha Salokannel | Nokia |
| Jari Syrjärinne | Nokia |
| Josselin de la Broise | Marvell |
| Joel Linsky | Qualcomm |
| Brian Redding | Qualcomm |
| Mayank Batra | Qualcomm Technologies, Inc. |
| Jean-Philippe Lambert | RivieraWaves |
| Rasmus Abildgren | Samsung Electronics |
| Clive D.W. Feather | Samsung Electronics |

# Contributors to Connection-Oriented AoA

| Name | Company |
| --- | --- |
| Jason Hillyard | ARM |
| Angel Polo | Broadcom |
| Mayank Batra | CSR |
| Ian Blair | CSR |
| Raja Banerjea | CSR |
| Sabita Nahata | CSR |
| Marcel Holtmann | Intel |
| Harish Balasubramaniam | Intel |
| Juha Salokannel | Nokia |
| Shwetha Mahadik | MindTree |
| Joel Linsky | Qualcomm Technologies, Inc. |
| Brian Redding | Qualcomm |
| Mayank Batra | Qualcomm Technologies, Inc. |
| Jean-Philippe Lambert | RivieraWaves |
| Rasmus Abildgren | Samsung Electronics |
| Clive D.W. Feather | Samsung Electronics |
| Florian Lefeuvre | Texas Instruments |

DISCLAIMER AND COPYRIGHT NOTICE

This disclaimer applies to all draft specifications and final specifications adopted by the Bluetooth SIG Board of Directors (both of which are hereinafter referred to herein as a Bluetooth “Specification”). Your use of this Specification in any way is subject to your compliance with all conditions of such use, and your acceptance of all disclaimers and limitations as to such use, contained in this Specification. Any user of this Specification is advised to seek appropriate legal, engineering or other professional advice regarding the use, interpretation or effect of this Specification on any matters discussed in this Specification.

Use of Bluetooth Specifications and any related intellectual property is governed by the Promoters Membership Agreement among the Promoter Members and Bluetooth SIG (the “Promoters Agreement”), certain membership agreements between Bluetooth SIG and its Adopter and Associate Members, including, but not limited to, the Membership Application, the Bluetooth Patent/Copyright License Agreement and the Bluetooth Trademark License Agreement (collectively, the “Membership Agreements”) and the Bluetooth Specification Early Adopters Agreements (1.2 Early Adopters Agreements) among Early Adopter members of the unincorporated Bluetooth SIG and the Promoter Members (the “Early Adopters Agreement”). Certain rights and obligations of the Promoter Members under the Early Adopters Agreements have been assigned to Bluetooth SIG by the Promoter Members.

Use of the Specification by anyone who is not a member of Bluetooth SIG or a party to an Early Adopters Agreement (each such person or party, a “Member”) is prohibited. The use of any portion of a Bluetooth Specification may involve the use of intellectual property rights ("IPR"), including pending or issued patents, or copyrights or other rights. Bluetooth SIG has made no search or investigation for such rights and disclaims any undertaking or duty to do so. The legal rights and obligations of each Member are governed by the applicable Membership Agreements, Early Adopters Agreement or Promoters Agreement. No license, express or implied, by estoppel or otherwise, to any intellectual property rights are granted herein.

Any use of the Specification not in compliance with the terms of the applicable Membership Agreements, Early Adopters Agreement or Promoters Agreement is prohibited and any such prohibited use may result in (i) termination of the applicable Membership Agreements or Early Adopters Agreement and (ii) liability claims by Bluetooth SIG or any of its Members for patent, copyright and/or trademark infringement claims permitted by the applicable agreement or by applicable law.

THE SPECIFICATION IS PROVIDED “AS IS” WITH NO WARRANTIES WHATSOEVER, INCLUDING ANY WARRANTY OF MERCHANTABILITY, NONINFRINGEMENT, FITNESS FOR ANY PARTICULAR PURPOSE, SATISFACTORY QUALITY, OR REASONABLE SKILL OR CARE, OR ANY WARRANTY ARISING OUT OF ANY COURSE OF DEALING, USAGE, TRADE PRACTICE, PROPOSAL, SPECIFICATION OR SAMPLE.

Each Member hereby acknowledges that products equipped with the Bluetooth wireless technology ("Bluetooth Products") may be subject to various regulatory controls under the laws and regulations applicable to products using wireless non licensed spectrum of various governments worldwide. Such laws and regulatory controls may govern, among other things, the combination, operation, use, implementation and distribution of Bluetooth Products. Examples of such laws and regulatory controls include, but are not limited to, airline regulatory controls, telecommunications regulations, technology transfer controls and health and safety regulations. Each Member is solely responsible for the compliance by their Bluetooth Products with any such laws and regulations and for obtaining any and all required authorizations, permits, or licenses for their Bluetooth Products related to such regulations within the applicable jurisdictions. Each Member acknowledges that nothing in the Specification provides any information or assistance in connection with securing such compliance, authorizations or licenses. NOTHING IN THE SPECIFICATION CREATES ANY WARRANTIES, EITHER EXPRESS OR IMPLIED, REGARDING SUCH LAWS OR REGULATIONS.

ALL LIABILITY, INCLUDING LIABILITY FOR INFRINGEMENT OF ANY INTELLECTUAL PROPERTY RIGHTS OR FOR NONCOMPLIANCE WITH LAWS, RELATING TO USE OF THE SPECIFICATION IS EXPRESSLY DISCLAIMED. To the extent not prohibited by law, in no event will Bluetooth SIG or its Members or their affiliates be liable for any damages, including without limitation, lost revenue, profits, data or programs, or business interruption, or for special, indirect, consequential, incidental or punitive damages, however caused and regardless of the theory of liability, arising out of or related to any furnishing, practicing, modifying, use or the performance or implementation of the contents of this Specification, even if Bluetooth SIG or its Members or their affiliates have been advised of the possibility of such damages. BY USE OF THE SPECIFICATION, EACH MEMBER EXPRESSLY WAIVES ANY CLAIM AGAINST BLUETOOTH SIG AND ITS MEMBERS OR THEIR AFFILATES RELATED TO USE OF THE SPECIFICATION.

If this Specification is an intermediate draft, it is for comment only. No products should be designed based on it except solely to verify the prototyping specification at SIG sponsored IOP events and it does not represent any commitment to release or implement any portion of the intermediate draft, which may be withdrawn, modified, or replaced at any time in the adopted Specification.

Bluetooth SIG reserves the right to adopt any changes or alterations to the Specification it deems necessary or appropriate.

Copyright © 2015. The Bluetooth word mark and logos are owned by Bluetooth SIG, Inc. All copyrights in the Bluetooth Specifications themselves are owned by Ericsson AB, Lenovo (Singapore) Pte. Ltd., Intel Corporation, Microsoft Corporation, Apple Inc., LLC, Nokia Corporation and Toshiba Corporation. Other third-party brands and names are the property of their respective owners.

# Contents

1	Change Overview	9

1.1	Major changes since the FIPDs	13

2	Change Request Instances	15

2.1	Volume 1, Part A (Architecture) Changes	15

2.1.1	[UPDATED SECTION] 1.2 OVERVIEW OF BLUETOOTH LOW ENERGY OPERATION	15

2.1.2	[UPDATED SECTION] 3.2.2 LE Generic Packet Structure	15

2.1.3	[UPDATED SECTION] 3.3.2 LE Physical Channels	15

2.1.4	[NEW SECTION] X Direction Finding using Bluetooth Low Energy	16

2.1.4.1	[NEW SECTION] X.1 Angle of Arrival (AoA) method	16

2.1.4.2	[NEW SECTION] X.2 Angle of Departure (AoD) method	18

2.2	Volume 1, Part B (Acronyms & Abbreviations) Changes	19

2.2.1	[UPDATED SECTION] 1 LIST OF ACRONYMS AND ABBREVIATIONS	19

2.3	Volume 2, Part E (HCI) Changes	20

2.3.1	[UPDATED SECTION] 3.4 CONTROLLER INFORMATION	21

2.3.2	[UPDATED SECTION] 3.5 CONTROLLER CONFIGURATION	21

2.3.3	[UPDATED SECTION] 3.6 DEVICE DISCOVERY	21

2.3.4	[UPDATED SECTION] 3.8 REMOTE INFORMATION	22

2.3.5	[UPDATED SECTION] 3.10 CONNECTION STATE	23

2.3.6	[UPDATED SECTION] 3.17 TESTING	24

2.3.7	[UPDATED SECTION] 3.18 ALPHABETICAL LIST OF COMMANDS AND EVENTS	24

2.3.8	[UPDATED SECTION] 3.19 LE CONTROLLER REQUIREMENTS	24

2.3.9	[UPDATED SECTION] 6.27 Supported Commands	26

2.3.10	[NEW SECTION] 7.7.65.XX LE Connectionless I&Q Report Event	27

2.3.11	[NEW SECTION] 7.7.65.XX+1 LE Connection I&Q Report Event	29

2.3.12	[UPDATED SECTION] 7.8.1 LE Set Event Mask Command	32

2.3.13	[NEW SECTION] 7.8.XX LE Set Supplemental Receive Test Parameters Command	32

2.3.14	[NEW SECTION] 7.8.XX+1 LE Set Supplemental Transmit Test Parameters Command	34

2.3.15	[NEW SECTION] 7.8.XX+2 LE Set Connectionless Supplemental Transmit Parameters Command	36

2.3.16	[NEW SECTION] 7.8.XX+3 LE Set Connectionless Supplemental Transmit Enable Command	38

2.3.17	[NEW SECTION] 7.8.XX+4 LE Read Connectionless I&Q Sampling List Size Command	39

2.3.18	[NEW SECTION] 7.8.XX+5 LE Add Device To Connectionless I&Q Sampling List Command	40

2.3.19	[NEW SECTION] 7.8.XX+6 LE Remove Device From Connectionless I&Q Sampling List Command	43

2.3.20	[NEW SECTION] 7.8.XX+7 LE Clear Connectionless I&Q Sampling List Command	45

2.3.21	[NEW SECTION] 7.8.XX+8 LE Set Connectionless I&Q Sampling Enable Command	46

2.3.22	[NEW SECTION] 7.8.XX+9 LE Set Connection Supplemental Receive Parameters Command	49

2.3.23	[NEW SECTION] 7.8.XX+10 LE Set Connection Supplemental Transmit Parameters Command	50

2.3.24	[NEW SECTION] 7.8.XX+11 LE Connection Supplemental Request Enable Command	52

2.3.25	[NEW SECTION] 7.8.XX+12 LE Connection Supplemental Response Enable Command	55

2.3.26	[NEW SECTION] 7.8.XX+13 LE Read Maximum Supplemental Length Command	56

2.4	Volume 3, Part C (Generic Access Profile) Changes	57

2.5	Volume 6, Part A (Physical Layer) Changes	57

2.5.1	[NEW SECTION] 4.X ANTENNA SWITCHING	57

2.6	Volume 6, Part B (Link Layer) Changes	58

2.6.1	[UPDATED SECTION] 2.1 PACKET FORMAT FOR LE UNCODED PHYS	58

2.6.2	[UPDATED SECTION] 2.1.4 CRC	59

2.6.3	[NEW SECTION] 2.1.X Supplemental	59

2.6.4	[NEW SECTION] 2.3.X.3 SuppInfo	59

2.6.5	[UPDATED SECTION] 2.4 DATA CHANNEL PDU	59

2.6.5.1	[UPDATED SECTION] 2.4.2 LL Control PDU	62

2.6.6	[UPDATED SECTION] 2.5 PACKET FORMAT FOR THE LE CODED PHY	63

2.6.7	[NEW SECTION] 2.X Supplemental and I&Q Sampling	63

2.6.7.1	[NEW SECTION] 2.X.1 Supplemental structure and types	63

2.6.7.2	[NEW SECTION] 2.X.2 SuppInfo field	64

2.6.7.3	[NEW SECTION] 2.X.3 Transmitting Supplementals	65

2.6.7.4	[NEW SECTION] 2.X.4 I&Q Sampling	65

2.6.8	[UPDATED SECTION] 3.2 DATA WHITENING	66

2.6.9	[UPDATED SECTION] 4.6 FEATURE SUPPORT	66

2.6.9.1	[NEW SECTION] 4.6.X Connection Supplemental Request	67

2.6.9.2	[NEW SECTION] 4.6.X+1 Connection Supplemental Response	67

2.6.9.3	[NEW SECTION] 4.6.X+2 Connectionless Supplemental Transmitter	67

2.6.9.4	[NEW SECTION] 4.6.X+3 Connectionless Supplemental Receiver	67

2.6.9.5	[NEW SECTION] 4.6.X+4 Antenna Switching During Supplemental Transmission (AoD)	68

2.6.9.6	[NEW SECTION] 4.6.X+5 Antenna Switching During Supplemental Reception (AoA)	68

2.6.10	[NEW SECTION] 5.1.X Supplemental Request Procedure	68

2.7	Volume 6, Part D (Message Sequence Charts) Changes	70

2.7.1	[NEW SECTION] 2.X Connectionless I&Q Sampling List	70

2.7.2	[NEW SECTION] 3.X Connectionless Supplemental transmission	70

2.7.3	[NEW SECTION] 4.X Connectionless Supplemental reception	71

2.7.4	[NEW SECTION] 6.X Supplemental Request	73

2.8	Volume 6, Part F (Direct Test Mode) Changes	75

2.8.1	[UPDATED SECTION] 3.3.2 Commands	75

2.8.2	[UPDATED SECTION] 3.4.1 LE_Test_Status_Event	76

2.8.3	[UPDATED SECTION] 4.1 LE Test Packet Format	77

2.8.4	[UPDATED SECTION] 4.1.4 LE Test Packet PDU	78

2.8.5	[NEW SECTION] 4.1.X Supplemental	78

3	References	80

Change Overview

| Section Number |  | New / Changed |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Volume 1, Part A (Architecture) Changes |  |  |  |  |  |
| 1.2 Overview of Bluetooth Low Energy Operation |  | Changed |  | AoA and AoD |  |
| 3.2.2 LE Generic Packet Structure |  | Changed |  | AoA and AoD |  |
| 3.3.2 LE Physical Channels |  | Changed |  | AoA and AoD |  |
| X Direction Finding using Bluetooth Low Energy |  | New |  | AoA and AoD |  |
| X.1 Angle of Arrival (AoA) method |  | New |  | AoA |  |
| X.2 Angle of Departure (AoD) method |  | New |  | AoD |  |
| Volume 1, Part B (Acronyms & Abbreviations) Changes |  |  |  |  |  |
| 1 LIST OF ACRONYMS AND ABBREVIATIONS |  | Changed |  | AoA and AoD |  |
| Volume 2, Part E (HCI) Changes |  |  |  |  |  |
| 3.4 CONTROLLER INFORMATION |  | Changed |  | AoA and AoD |  |
| 3.5 CONTROLLER CONFIGURATION |  | Changed |  | AoD |  |
| 3.6 DEVICE DISCOVERY |  | Changed |  | AoD |  |
| 3.8 REMOTE INFORMATION | Changed | AoD |
| 3.10 CONNECTION STATE |  | Changed |  | AoA |  |
| 3.17 TESTING | Changed | AoA and AoD |
| 3.18 ALPHABETICAL LIST OF COMMANDS AND EVENTS | Changed | AoA and AoD |
| 3.19 LE CONTROLLER REQUIREMENTS |  | Changed |  | AoA and AoD |  |
| 6.27 Supported Commands |  | Changed |  | AoA and AoD |  |
| 7.7.65.XX LE Connectionless I&Q Report Event |  | New |  | AoD |  |
| 7.7.65.XX+1 LE Connection I&Q Report Event |  | New |  | AoA |  |
| 7.8.1 LE Set Event Mask Command |  | Changed |  | AoA and AoD |  |
| 7.8.XX LE Set Supplemental Receive Test Parameter Command |  | New |  | AoA and AoD |  |
| 7.8.XX+1 LE Set Supplemental Transmit Test Parameters Command |  | New |  | AoA and AoD |  |
| 7.8.XX+2 LE Set Connectionless Supplemental Transmit Parameters Command |  | New |  | AoD |  |
| 7.8.XX+3 LE Set Connectionless Supplemental Transmit Enable Command |  | New |  | AoD |  |
| 7.8.XX+4 LE Read Connectionless I&Q Sampling List Size Command |  | New |  | AoD |  |
| 7.8.XX+5 LE Add Device To Connectionless I&Q Sampling List Command |  | New |  | AoD |  |
| 7.8.XX+6 LE Remove Device From Connectionless I&Q Sampling List Command |  | New |  | AoD |  |
| 7.8.XX+7 LE Clear Connectionless I&Q Sampling List Command |  | New |  | AoD |  |
| 7.8.XX+8 LE Set Connectionless I&Q Sampling Enable Command |  | New |  | AoD |  |
| 7.8.XX+9 LE Set Connection Supplemental Receive Parameters Command |  | New |  | AoA |  |
| 7.8.XX+10 LE Set Connection Supplemental Transmit Parameters Command |  | New |  | AoA |  |
| 7.8.XX+11 LE Connection Supplemental Request Enable Command |  | New |  | AoA |  |
| 7.8.XX+12 LE Connection Supplemental Response Enable Command |  | New |  | AoA |  |
| 7.8.XX+13 LE Read Maximum Supplemental Length Command |  | New |  | AoA and AoD |  |
| Volume 3, Part C (Generic Access Profile) Changes |  |  |
| Volume 6, Part A (Physical Layer) Changes |  |  |  |  |  |
| 4.X ANTENNA SWITCHING |  | New |  | AoA and AoD |  |
| Volume 6, Part B (Link Layer) Changes |  |  |  |  |  |
| 2.1 PACKET FORMAT |  | Changed |  | AoA and AoD |  |
| 2.1.4 CRC |  | Changed |  | AoA and AoD |  |
| 2.1.X Supplemental |  | New |  | AoA and AoD |  |
| 2.3.X.3 SuppInfo |  | New |  | AoD |  |
| 2.4 DATA CHANNEL PDU |  | Changed |  | AoA |  |
| 2.4.2 LL Control PDU |  | Changed |  | AoA |  |
| 2.4.2.X LL_SUPPLEMENTAL_REQ |  | New |  | AoA |  |
| 2.4.2.X+1 LL_SUPPLEMENTAL_RSP |  | New |  | AoA |  |
| 2.5 PACKET FORMAT FOR THE LE CODED PHY |  | Changed |  | AoA and AoD |  |
| 2.X Supplemental and I&Q Sampling |  | New |  | AoA and AoD |  |
| 2.X.1 Supplemental structure and types |  | New |  | AoA and AoD |  |
| 2.X.2 SuppInfo field |  | New |  | AoA and AoD |  |
| 2.X.3 (Transmitting Supplementals) |  | New |  | AoA and AoD |  |
| 2.X.4 I&Q Sampling |  | New |  | AoA and AoD |  |
| 3.2 DATA WHITENING |  | Changed |  | AoA and AoD |  |
| 4.6 FEATURE SUPPORT |  | Changed |  | AoA and AoD |  |
| 4.6.X Connection Supplemental Request |  | New |  | AoA |  |
| 4.6.X+1 Connection Supplemental Response |  | New |  | AoA |  |
| 4.6.X+2 Connectionless Supplemental Transmitter |  | New |  | AoD |  |
| 4.6.X+3 Connectionless Supplemental Receiver |  | New |  | AoD |  |
| 4.6.X+4 Antenna Switching During Supplemental Transmission (AoD) |  | New |  | AoD |  |
| 4.6.X+5 Antenna Switching During Supplemental Reception (AoA) |  | New |  | AoA |  |
| 5.1.X Supplemental Request Procedure |  | New |  | AoA |  |
| Volume 6, Part D (Message Sequence Charts) Changes |  |  |  |  |  |
| 2.X Connectionless I&Q Sampling List |  | New |  | AoD |  |
| 3.X Connectionless Supplemental transmission |  | New |  | AoD |  |
| 4.X Connectionless Supplemental reception |  | New |  | AoD |  |
| 6.X Supplemental Request |  | New |  | AoA |  |
| Volume 6, Part F (Direct Test Mode) Changes |  |  |  |  |  |
| 3.3.2 Commands |  | Changed |  | AoA and AoD |  |
| 3.4.1 LE_Status_Event |  | Changed |  | AoA and AoD |  |
| 4.1 LE Test Packet Format |  | Changed |  | AoA and AoD |  |
| 4.1.4 LE Test Packet PDU |  | Changed |  | AoA andAoD |  |
| 4.1.X Supplemental |  | New |  | AoA and AoD |  |

Table 1.1 Change Overview table

Major changes since the FIPDs

- Section 3 of HCI (Overview of Commands and Events) has been completed.

- HCI commands and events have been renamed and their formats have been changed in order to make them more generic. This would help in the future in case we decide to introduce Connectionless AoA and/or Connection-Oriented AoD.

- In the Data Channel PDU header, the Supplemental Time (ST) field has been replaced with SuppInfo. Thus the Supplemental Time Present (STP) bit is now called SuppInfo Present (SP) bit.

- The LL_AOA_REQ and LL_AOA_RSP PDUs are now called LL_SUPPLEMENTAL_REQ and LL_SUPPLEMENTAL_RSP PDUs.

- In the I&Q Sampling for Angle of Arrival section, it is no longer required to use the same antenna used for transmitting the body of the packet to transmit the Supplemental during the guard period and the reference period.

- Feature bits have been allocated to Connectionless Supplemental Transmitter, Connectionless Supplemental Receiver, Antenna Switching During Supplemental Transmission and Reception.

- The AoA Procedure is now called Supplemental Request Procedure.

- Direct Test Mode section has been completed.

- Test Mode HCI commands have been updated in order to support transmitting and receiving a Supplemental.

- In the case of periodic operation, the Supplemental Request Interval is now measured in number of connection intervals instead of milliseconds.

- A new HCI command has been added to read the maximum Supplemental length supported by the Controller.

- The LL_SUPPLEMENTAL_REQ PDU now also contains a field to indicate the type of Supplemental that the remote device should send.

- In the SuppInfo field, the SupplementalTime is now 5 bits instead of 6 bits. As a result 1 bit  has been reserved for future use.

- Message sequence charts have been added.

- In the Test Mode packet header a bit has been allocated to indicate the presence of SuppInfo in the header and the presence of the Supplemental in the packet.

- The reference example has been removed. It will be added to the profile spec(s).

- Supported Commands (HCI Section 6.27) has been updated.

- The LE Connectionless I&Q Report event is generated in Test Mode.

- Issue 7084 changes have been applied.

- Bring up to date with the latest LE Advertising Extension CR.

- Two new test mode HCI commands: one to set the parameters for transmitting the Supplemental and the other to set the parameters for receiving the Supplemental during test modes.

Change Request Instances

Volume 1, Part A (Architecture) Changes

[UPDATED SECTION] 1.2 OVERVIEW OF BLUETOOTH LOW ENERGY OPERATION

[Insert at the end of the section]

The LE radio provides a means for detecting the relative direction of another LE radio by using the Angle of Arrival (AoA) or Angle of Departure (AoD) method.

[UPDATED SECTION] 3.2.2 LE Generic Packet Structure

[Make one change to figure 3.5 as shown below. Rest of the figure is unchanged]

# Figure 3.5: LE packet structure

[Modify the last paragraph as follows]

For data channel PDUs, the PDU header contains the Logical Link Identifier (LLID), the Next Expected Sequence Number (NESN), Sequence Number (SN), More Data (MD), SuppInfo Present (SP), and payload length, and may contain SuppInfo. For data channel PDUs that contain control commands, the data channel PDU payload contains a command opcode and control data that is specific to the command. There is an optional Message Integrity Check (MIC) value that is used to authenticate the data PDU. For data channel PDUs that are data, the data channel PDU payload contains L2CAP data.

[Insert at the end of the section]

Both advertising channel packets and data channel packets may contain a Supplemental, which can be used for determining the relative direction of a received radio signal.

[UPDATED SECTION] 3.3.2 LE Physical Channels

[Insert at the end of the section]

Packets on both the LE piconet physical channel and the LE advertisement broadcast channel may carry a Supplemental that can be used for the purpose of direction finding.

[NEW SECTION] X Direction Finding using Bluetooth Low Energy

An LE device can make its direction available for a peer device by transmitting direction finding enabled packets. Using direction information from several transmitters and profile-level information giving their locations, an LE radio can calculate its own position.

This facility is supported over the LE Uncoded PHYs, but not over the LE Coded PHY.

[NEW SECTION] X.1 Angle of Arrival (AoA) method

An LE device can make its direction available to a peer device by transmitting direction finding enabled packets using a single antenna.

The peer device, consisting of an RF switch and antenna array, switches antennae while receiving part of those packets and captures I&Q samples. The I&Q samples can be used to calculate the phase difference in the radio signal received using different elements of the antenna array which in turn can be used to estimate the angle of arrival (AoA).

[Note to the editor: on the Transmitter side:

- Change “AoA Enhanced Controller” to “LE Transmitter”.

- Remove the word “Transmitter”.

On the Receiver side:

- Change “AoA Enhanced Controller” to “LE Receiver”.

- Remove the word “Receiver”.

- Below the “AoA Enhanced Controller” box (now changed to “LE Receiver” box), draw a box labelled “AoA Estimation” and draw an arrow from the “LE Receiver” box to the “AoA Estimation” box.]

# Figure X.1: Angle of Arrival method

Consider a receiver device with an antenna array consisting of two antennae, separated by distance d. The transmitter device uses a single antenna to transmit a signal. As shown in Figure X.2, a perpendicular line can be drawn from an incoming signal wave front extending to the furthest antenna (antenna 2) at the point of intersection to the closest antenna (antenna 1). The adjacent side of that right triangle represents the path difference relative to the angle of incidence of that wave front between both antennae. The phase difference, ψ, in the signal arriving at the two antennae is then

[Note to the editor: put the following two equations into equation format. Also ensure that ‘d’ and ‘ψ’ in the paragraph above are in the same style.]

ψ = (2πd cos(θ))/λ

where λ is the wavelength of the signal and θ is the AoA, and so

θ = cos-1((ψλ)/(2πd))

# Figure X.2 Measuring the Angle of Arrival

[NEW SECTION] X.2 Angle of Departure (AoD) method

A device consisting of an RF switch and antenna array can make its angle of departure (AoD) detectable by transmitting direction finding enabled packets, switching antennae during transmission.

The peer device receives those packets using a single antenna and captures I&Q samples during part of those packets. Determination of the direction is based on the different propagation delays of the LE radio signal between the transmitting elements of the antenna array and a receiving single antenna. The propagation delays are detectable with I&Q measurements. Any receiving LE radio with a single antenna that supports the AoD feature can capture I&Q samples and, with the aid of profile-level information specifying the antenna layout of the transmitter, calculate the angle of incidence of the incoming radio signal.

# Figure X.3: Angle of Departure method

Consider a transmitter device with an antenna array consisting of two antennae, separated by distance d. The receiver device uses a single antenna to receive the signals. The phase difference, ψ, in the signal from antenna 1 and the signal from antenna 2 arriving at the receiver is then

[Note to the editor: put the following two equations into equation style. Also ensure that ‘d’ and ‘ψ’ in the paragraph above are in the same style.]

ψ = (2πd sin(θ))/λ

where λ is the wavelength of the signal and θ is the AoD, and so

θ = sin-1((ψλ)/(2πd))

# Figure X.4 Measuring the Angle of Departure

Volume 1, Part B (Acronyms & Abbreviations) Changes

[UPDATED SECTION] 1 LIST OF ACRONYMS AND ABBREVIATIONS

[Add the new rows in red in proper alphabetical places in table 1.1]

| Acronym or abbreviation | Writing out in full | Comments |
| --- | --- | --- |
| AoA | Angle of Arrival |  |
| AoD | Angle of Departure |  |
| I&Q | In-phase and Quadrature |  |
| SuppInfo | Supplemental Information | A field in the data channel PDU Header and the extended advertising header. |
| SP | SuppInfo Present | A field in the data channel PDU Header to indicate the presence of the SuppInfo field. |

Table 1.1: Acronyms and Abbreviations

Volume 2, Part E (HCI) Changes

[Note that the following terms have been defined in the LE Advertising Extension CR [2]:

- Advertising_Handle

- Advertising_SID

- LE Set Extended Advertising Enable command

- LE Set Extended Advertising Parameters command

- LE Set Periodic Advertising Parameters command

- LE Set Extended Scan Enable command

- LE Set Extended Scan Parameters command

- Sync_Handle

]

[UPDATED SECTION] 3.4 CONTROLLER INFORMATION

| Name | Vers. | Summary description | Supported Controllers |
| --- | --- | --- | --- |
| LE Read Maximum Supplemental Length Command | Madrid | The LE Read Maximum Supplemental Length command allows the Host to read the maximum length of the Supplemental supported by the Controller. | LE |

Table 3.5: Controller information

[UPDATED SECTION] 3.5 CONTROLLER CONFIGURATION

| Name | Vers. | Summary description | Supported Controllers |
| --- | --- | --- | --- |
| LE Set Connectionless Supplemental Transmit Parameters Command | Madrid | The LE Set Connectionless Supplemental Transmit Parameters Command will set the antenna-switching pattern, and switching and sampling slot durations for the transmission of the Supplemental. | LE |
| LE Set Connectionless Supplemental Transmit Enable Command | Madrid | The LE Set Connectionless Supplemental Transmit Enable Command will request the Controller to enable or disable sending packets containing the Supplemental. | LE |
| LE Set Connectionless I&Q Sampling Receive Enable | Madrid | The LE Set Connectionless I&Q Sampling Enable Command will request the Controller to enable or disable taking I&Q samples from the Supplementals of advertising packets. | LE |

Table 3.6: Controller configuration (Sheet 4 of 4)

[UPDATED SECTION] 3.6 DEVICE DISCOVERY

| Name | Vers. | Summary description | Supported Controllers |  |
| --- | --- | --- | --- | --- |

[UPDATED SECTION] 3.10 CONNECTION STATE

| Name | Vers. | Summary description | Supported Controllers |  |
| --- | --- | --- | --- | --- |
|  |  | - |  |
|  |  | - |  |

| LE Read Connectionless I&Q Sampling List Size Command |  | Madrid |  | The LE Read Connectionless I&Q Sampling List Size Command will read the maximum number of I&Q sampling list entries that the Controller supports. |  | LE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LE Add Device To Connectionless I&Q Sampling List Command |  | Madrid |  | The LE Add Device To Connectionless I&Q Sampling List Command will add a device to the list of advertisers from which the Controller is requested to take I&Q samples. |  | LE |  |  |
| LE Remove Device From Connectionless I&Q Sampling List Command |  | Madrid |  | The LE Remove Device From Connectionless I&Q Sampling List Command will remove a single device from the list of advertisers from which the Controller is requested to take I&Q samples. |  | LE |  |  |
| LE Clear Connectionless I&Q Sampling List Command |  | Madrid |  | The LE Clear Connectionless I&Q Sampling List Command will clear the list of advertiser addresses stored for I&Q sampling in the Controller. |  | LE |  |  |
|  |  | samples |  |  |

Table 3.7: Device discovery (Sheet 3 of 3)

[UPDATED SECTION] 3.8 REMOTE INFORMATION

| Name | Vers. | Summary description | Supported Controllers |
| --- | --- | --- | --- |
| LE Connectionless I&Q Report Event | Madrid | The LE Connectionless I&Q Report Event reports I&Q information from the Supplemental of a received advertising packet. | LE |

Table 3.9: Remote information

[UPDATED SECTION] 3.10 CONNECTION STATE

| Name | Vers. | Summary description | Supported Controllers |
| --- | --- | --- | --- |
| LE Set Connection Supplemental Receive Parameters Command | Madrid | The LE Set Connection Supplemental Receive Parameters Command will set the antenna-switching pattern, switching and sampling slot durations for receiving Supplementals on a connection. | LE |
| LE Set Connection Supplemental Transmit Parameters Command | Madrid | The LE Set Connection Supplemental Transmit Parameters Command will set the antenna-switching pattern, switching and sampling slot durations for transmitting Supplementals on a connection. | LE |
| LE Connection Supplemental Request Enable Command | Madrid | The LE Connection Supplemental Request Enable Command will request the Controller to start or stop sending of LL_SUPPLEMENTAL_REQ PDUs on a connection. | LE |
| LE Connection Supplemental Response Enable Command | Madrid | The LE Connection Supplemental Response Enable Command will command the Controller to respond to LL_SUPPLEMENTAL_REQ PDUs with LL_SUPPLEMENTAL_RSP PDUs. | LE |
| LE Connection I&Q Report Event | Madrid | The LE Connection I&Q Report Event is used to report I&Q samples from the Supplemental of a received LL_SUPPLEMENTAL_RSP PDU. | LE |

Table 3.11: Connection state

[UPDATED SECTION] 3.17 TESTING

| Name | Vers. | Summary description | Supported Controllers |
| --- | --- | --- | --- |
| LE Set Supplemental Receive Test Parameters Command | Madrid | The LE Set Supplemental Receive Test Parameters command will set the antenna switching pattern and the switching and sampling slot durations for receiving Supplementals during test mode. | LE |
| LE Set Supplemental Transmit Test Parameters Command | Madrid | The LE Set Supplemental Transmit Test Parameters command is used to set the antenna switching pattern and the switching slot durations for transmitting Supplementals during test mode. | LE |

Table 3.18: Testing (Sheet 2 of 2)

[UPDATED SECTION] 3.18 ALPHABETICAL LIST OF COMMANDS AND EVENTS

[Note to the editor: add all new commands and events to Table 3.19]

[UPDATED SECTION] 3.19 LE CONTROLLER REQUIREMENTS

| Name | LE Feature Requirements |  |
| --- | --- | --- |
| LE Set Connection Supplemental Receive Parameters Command |  | CTBD1 |  |  |
| LE Connection Supplemental Request Enable Command |  | CTBD1 |  |  |
| LE Connection I&Q Report Event |  | CTBD1 |  |  |
| LE Set Connection Supplemental Transmit Parameters Command |  | CTBD2 |  |  |
| LE Connection Supplemental Response Enable Command |  | CTBD2 |  |  |
| LE Set Connectionless Supplemental Transmit Parameters Command |  | CTBD3 |  |  |
| LE Set Connectionless Supplemental Transmit Enable Command |  | CTBD3 |  |  |
| LE Read Connectionless I&Q Sampling List Size Command |  | CTBD4 |  |  |
| LE Add Device To Connectionless I&Q Sampling List Command |  | CTBD4 |  |  |
| LE Remove Device From Connectionless I&Q Sampling List Command |  | CTBD4 |  |  |
| LE Clear Connectionless I&Q Sampling List Command |  | CTBD4 |  |  |
| LE Set Connectionless I&Q Sampling Enable Command |  | CTBD4 |  |  |
| LE Connectionless I&Q Report Event |  | CTBD4 |  |  |
| LE Read Maximum Supplemental Length Command |  | CTBD5 |  |  |
| LE Set Supplemental Transmit Test Parameters Command | CTBD5 |
| LE Set Supplemental Receive Test Parameters Command | CTBD6 |

Table 3.20: Bluetooth Controller supporting LE requirements

CTBD1: Mandatory if LE Feature (Connection Supplemental Request) is supported, otherwise excluded.

CTBD2: Mandatory if LE Feature (Connection Supplemental Response) is supported, otherwise excluded.

CTBD3: Mandatory if LE Feature (Connectionless Supplemental Transmitter) is supported, otherwise excluded.

CTBD4: Mandatory if LE Feature (Connectionless Supplemental Receiver) is supported, otherwise excluded.

CTBD5: Mandatory if LE Feature (Connection Supplemental Response) or LE Feature (Connectionless Supplemental Transmitter) is supported, otherwise excluded.

CTBD6: Mandatory if LE Feature (Connection Supplemental Request) or LE Feature (Connectionless Supplemental Receiver) is supported, otherwise excluded.

[UPDATED SECTION] 6.27 Supported Commands

| Octet | Bit | Command Supported |
| --- | --- | --- |
| TBD | TBD | LE Set Supplemental Receive Test Parameters |
| TBD | TBD | LE Set Supplemental Transmit Test Parameters |
| TBD | TBD | LE Set Connectionless Supplemental Transmit Parameters |
| TBD | TBD | LE Set Connectionless Supplemental Transmit Enable |
| TBD | TBD | LE Read Connectionless I&Q Sampling List Size |
| TBD | TBD | LE Add Device To Connectionless I&Q Sampling List |
| TBD | TBD | LE Remove Device From Connectionless I&Q Sampling List |
| TBD | TBD | LE Clear Connectionless I&Q Sampling List |
| TBD | TBD | LE Set Connectionless I&Q Sampling Enable |
| TBD | TBD | LE Set Connection Supplemental Receive Parameters |
| TBD | TBD | LE Set Connection Supplemental Transmit Parameters |
| TBD | TBD | LE Connection Supplemental Request Enable |
| TBD | TBD | LE Connection Supplemental Response Enable |
| TBD | TBD | LE Read Maximum Supplemental Length Command |

[NEW SECTION] 7.7.65.XX LE Connectionless I&Q Report Event

| Event | Event Code | Event Parameters |
| --- | --- | --- |
| LE Connectionless I&Q Report | 0x3E | Subevent_Code, Sync_Handle, Channel_Index, Supplemental_Type, Slot_Durations, Sample_Count, I_Sample[i], Q_Sample[i] |

# Description:

This event is used by the Controller to report I&Q information from the Supplemental of a received advertising packet containing the AUX_SYNC_IND PDU (see [Vol 6] Part B Section 2.3.1.X) from an advertiser identified by Sync_Handle.

The index of the channel on which the AUX_SYNC_IND advertising PDU was received, the type of Supplemental, and the I&Q samples of the Supplemental of the advertisement are reported in the corresponding parameters. The  parameter specifies the sampling rate used by the Controller.

The Supplemental format is defined in [Vol 6] Part B, Section 2.X.1.

If the AUX_SYNC_IND PDU contains AdvData, then the LE Extended Advertising Report event shall be generated before this event.

This event is also used by the Controller to report I&Q information from the Supplemental of a received Test Mode packet (see Section 7.8.50).

# Event Parameters:

| Subevent_Code: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| TBD | Subevent code for LE Connectionless I&Q Report event |  |

| Sync_Handle: |  | Size: 2 Octets (12 bits meaningful) |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXXXX | Sync_Handle to be used to identify the periodic advertiser. Range: 0x0000 – 0x0EFF (0x0F00 – 0x0FFE Reserved for future use) |  |
| 0x0FFF | Receiver Test |  |

| Channel_Index: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 – 0x24 | The index of the channel on which the AUX_SYNC_IND PDU was received. |  |
| All other values | Reserved for future use |  |

| Supplemental_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | AoA Supplemental |  |
| 0x01 | AoD Supplemental with 1 µs slots |  |
| 0x02 | AoD Supplemental with 2 µs slots |  |
| All other values | Reserved for future use |  |

| Slot_Durations: |  |  |  | Size: 1 Octet |
| --- | --- | --- | --- | --- |
| Value | Parameter Description |  |
| 0x01 |  | 1 µs sample slots |  |  |
| 0x02 |  | 2 µs sample slots |  |  |
| All other values |  | Reserved for future use |  |  |

| Sample_Count: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x09 – 0x52 |  | Total number of sample pairs (there shall be the same number of I samples and Q samples). Note: This number is dependent on the switch and sample slot durations used. |  |  |
| All other values |  | Reserved for future use |  |  |

| I_Sample [i]: |  | Size: Sample_Count * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | The list of I samples for the reported AUX_SYNC_IND PDU. Each sample is a signed integer. The list is in the order of the sampling points within the AUX_SYNC_IND PDU. |  |

| Q_Sample [i]: |  | Size: Sample Count * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | The list of Q samples for the reported AUX_SYNC_IND PDU. Each sample is a signed integer. The list is in the order of the sampling points within the AUX_SYNC_IND PDU. |  |

[NEW SECTION] 7.7.65.XX+1 LE Connection I&Q Report Event

| Event | Event Code | Event Parameters |
| --- | --- | --- |
| LE Connection I&Q Report | 0x3E | Subevent_Code, Status, Connection_Handle, RX_PHY, Data_Channel_Index, RSSI, Supplemental_Type, Slot_Durations, Sample_Count, I_Sample[i], Q_Sample[i] |

# Description:

This event is used by the Controller to report the I&Q samples from the Supplemental of a received LL_SUPPLEMENTAL_RSP PDU (see [Vol 6] Part B Section 2.4.2.X+1).

The Connection_Handle parameter identifies the connection that corresponds to the reported information.

The receiver PHY, the index of the data channel, the RSSI value (if available), the type of Supplemental, and the I&Q samples of the Supplemental of the received LL_SUPPLEMENTAL_RSP are reported in the corresponding parameters. The Slot_Durations parameter specifies the sampling rate used by the Controller.

The Supplemental format is defined in [Vol 6] Part B, Section 2.X.1.

# Event Parameters:

| Subevent_Code: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| TBD | Subevent code for LE Connection I&Q Report event |  |

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LL_SUPPLEMENTAL_RSP PDU received successfully. |  |
| 0x01 – 0xFF | LL_SUPPLEMENTAL_RSP PDU not received successfully. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

| Connection_Handle: |  | Size: 2 Octets (12 bits meaningful) |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXXXX | Connection_Handle Range 0x0000 – 0x0EFF (0x0F00 – 0x0FFF Reserved for future use) |  |

| RX_PHY: |  | Size: 1 Octet |
| --- | --- | --- |
| Bit number | Meaning |  |
| 0 | The receiver PHY for the connection is LE 1M |  |
| 1 | The receiver PHY for the connection is LE 2M |  |
| 2 – 7 | Reserved for future use |  |

| Data_Channel_Index: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 – 0x24 | The index of the data channel on which the data channel PDU was received. |  |
| All other values | Reserved for future use |  |

| RSSI: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| N | Size: 1 Octet (signed integer) Range: -127 ≤ N ≤ +20 Units: dBm |  |
| 21 – 126 | Reserved for future use |  |
| 127 | RSSI is not available |  |

| Supplemental_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | AoA Supplemental |  |
| 0x01 | AoD Supplemental with 1 µs slots |  |
| 0x02 | AoD Supplemental with 2 µs slots |  |
| All other values | Reserved for future use |  |

| Slot_Durations: |  |  |  | Size: 1 Octet |
| --- | --- | --- | --- | --- |
| Value | Parameter Description |  |
| 0x01 |  | 1 µs sample slots |  |  |
| 0x02 |  | 2 µs sample slots |  |  |
| All other values |  | Reserved for future use |  |  |

| Sample_Count: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x09 – 0x52 | Total number of sample pairs (there shall be the same number of I samples and Q samples). Note: This number is dependent on the switch and sample slot durations used. |  |
| All other values | Reserved for future use |  |

| I_Sample [i]: |  | Size: Sample_Count * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | The list of I samples for the reported PDU. Each sample is a signed integer. The list is in the order of the sampling points within the PDU. |  |

| Q_Sample [i]: |  | Size: Sample_Count * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | The list of Q samples for the reported PDU. Each sample is a signed integer. The list is in the order of the sampling points within the PDU. |  |

[UPDATED SECTION] 7.8.1 LE Set Event Mask Command

# Command Parameters:

LE_Event_Mask:								Size: 8 Octets

| Bit | LE Subevent Types |
| --- | --- |
| TBD | LE Connectionless I&Q Report Event |
| TBD | LE Connection I&Q Report Event |

[NEW SECTION] 7.8.XX LE Set Supplemental Receive Test Parameters Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Set_Supplemental_Receive_Test_Parameters | TBD | Supplemental_Length, Supplemental_Slot_Type, Number_of_Antennae, Antenna_Switching_Pattern | Status |

# Description:

The LE_Set_Supplemental_Receive_Test_Parameters command is used to set the antenna switching pattern and the switching and sampling slot durations for receiving Supplementals during test mode.

If the Supplemental_Slot_Type parameter is set to 0x01 and the Controller does not support 1 µs switching and sampling, the Controller shall return the error code Unsupported Feature or Parameter value (0x11).

The Supplemental_Slot_Type parameter is only valid when receiving an AoA Supplemental.

To test AoD receiving, Number_of_Antennae and Antenna_Switching_Pattern shall both be 0x00. To test AoA receiving, both shall be non-zero.

If the length of the Supplemental in a received test reference packet is not equal to Supplemental_Length then the DUT shall discard that packet.

# Command Parameters:

| Supplemental_Length: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | No Supplemental expected |  |
| 0x02 – 0x14 | Expected length of the Supplemental in 8 µs units |  |
| All other values | Reserved for future use |  |

| Supplemental_Slot_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | No Supplemental |  |
| 0x01 | Sample Supplemental with 1 µs slots |  |
| 0x02 | Sample Supplemental with 2 µs slots |  |
| All other values | Reserved for future use |  |

| Number_of_Antennae: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Antenna array not specified |  |
| 0x01 – 0x4B | Number of antennae in the antenna array |  |
| All other values | Reserved for future use |  |

| Antenna_Switching_Pattern: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Antenna switching pattern not specified |  |
| 0x01 | Antenna switching pattern A (see [Vol 6] Part F, Section 3.3.2) |  |
| 0x02 | Antenna switching pattern B (see [Vol 6] Part F, Section 3.3.2) |  |
| All other values | Reserved for future use |  |

Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Set_Supplemental_Receive_Test_Parameters command succeeded. |  |
| 0x01 – 0xFF | LE_Set_Supplemental_Receive_Test_Parameters command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Set_Supplemental_Receive_Test_Parameters command has completed, a Command Complete event shall be generated.

If the Supplemental_Length parameter is not set to zero, then LE Connectionless I&Q Report events may be generated by the Controller.

[NEW SECTION] 7.8.XX+1 LE Set Supplemental Transmit Test Parameters Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Set_Supplemental_Transmit_Test_Parameters | TBD | Supplemental_Length, Supplemental_Slot_Type, Number_of_Antennae, Antenna_Switching_Pattern | Status |

# Description:

The LE_Set_Supplemental_Transmit_Test_Parameters command is used to set the antenna switching pattern and the switching slot durations for transmitting Supplementals during test mode.

If the Supplemental_Slot_Type parameter is set to 0x01 and the Controller does not support 1 µs sampling, the Controller shall return the error code Unsupported Feature or Parameter value (0x11).

To transmit an AoA Supplemental, Number_of_Antennae and Antenna_Switching_Pattern shall both be 0x00. To transmit an AoD Supplemental, both shall be non-zero.

# Command Parameters:

| Supplemental_Length: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Do not transmit a Supplemental |  |
| 0x02 – 0x14 | Length of the Supplemental in 8 µs units |  |
| All other values | Reserved for future use |  |

| Supplemental_Slot_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | No switching during Supplemental |  |
| 0x01 | Switch Supplemental with 1 µs slots |  |
| 0x02 | Switch Supplemental with 2 µs slots |  |
| All other values | Reserved for future use |  |

| Number_of_Antennae: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Antenna array not specified |  |
| 0x01 – 0x4B | Number of antennae in the antenna array |  |
| All other values | Reserved for future use |  |

| Antenna_Switching_Pattern: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Antenna switching pattern not specified |  |
| 0x01 | Antenna switching pattern A (see [Vol 6] Part F, Section 3.3.2) |  |
| 0x02 | Antenna switching pattern B (see [Vol 6] Part F, Section 3.3.2) |  |
| All other values | Reserved for future use |  |

Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Set_Supplemental_Transmit_Test_Parameters command succeeded. |  |
| 0x01 – 0xFF | LE_Set_Supplemental_Transmit_Test_Parameters command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Set_Supplemental_Transmit_Test_Parameters command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+2 LE Set Connectionless Supplemental Transmit Parameters Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Set_Connectionless_Supplemental_Transmit_Parameters | TBD | Advertising_Handle, Number_of_Antenna_IDs, Antenna_IDs[i], Supplemental_Type, Supplemental_Length | Status |

Description:

The LE_Set_Connectionless_Supplemental_Transmit_Parameters command is used to set the antenna switching pattern and switching and sampling slot durations for the transmission of the Supplemental of Advertising PDUs for the advertising set identified by the Advertising_Handle parameter. The command is also used to set the Supplemental length and type.

This command shall be issued at least once before the LE_Set_Connectionless_Supplemental_Enable command is first issued for that advertising set; otherwise the Controller shall return the Command Disallowed (0x0C) error code.

If the Host issues this command when Supplementals have been enabled in the advertising set, the Controller shall return the error code Command Disallowed (0x0C).

If the Supplemental_Type parameter is set to 0x01 and the Controller does not support 1 µs switching slots, the Controller shall return the error code Unsupported Feature or Parameter value (0x11).

If the advertising set corresponding to the Advertising_Handle parameter does not exist, the Controller shall return the error code Unknown Advertising Identifier (0x42).

# Command Parameters:

| Advertising_Handle: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 – 0xEF | Used to identify an advertising set. |  |
| All other values | Reserved for future use |  |

| Number_of_Antenna_IDs: |  |  | Size: 1 Octet |
| --- | --- | --- | --- |
| Value | Parameter Description |  |
| 0x01 – 0x4B | The number of Antenna IDs in the sequence |  |  |
| All other values | Reserved for future use |  |  |

| Antenna_IDs[i]: |  | Size: Number_of_Antenna_IDs * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | List of Antenna IDs in the sequence. The Antenna IDs shall take values from 0 to Number_of_Antenna_IDs-1 inclusive. |  |

| Supplemental_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | AoA Supplemental |  |
| 0x01 | AoD Supplemental with 1 µs slots |  |
| 0x02 | AoD Supplemental with 2 µs slots |  |
| All other values | Reserved for future use |  |

| Supplemental_Length: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | No Supplemental |  |
| 0x02 – 0x14 | Supplemental length in 8 µs units |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Set_Connectionless_Supplemental_Transmit_Parameters command succeeded. |  |
| 0x01 – 0xFF | LE_Set_Connectionless_Supplemental_Transmit_Parameters command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Set_Connectionless_Supplemental_Transmit_Parameters command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+3 LE Set Connectionless Supplemental Transmit Enable Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Set_Connectionless_Supplemental_Transmit_Enable | TBD | Advertising_Handle, Supplemental_Enable | Status |

# Description:

The LE_Set_Connectionless_Supplemental_Transmit_Enable command is used to request that the Controller enables or disables sending packets containing the AUX_SYNC_IND PDU and the SupplementalType field of the SuppInfo set to value “AoD Supplemental” on the advertising set identified by Advertising_Handle.

The Host must also enable advertising using the LE_Set_Extended_Advertising_Enable command (see Section 7.8.57).

The Host shall create the advertising set using the LE_Set_Periodic_Advertising_Parameters command (see Section 7.8.62) before issuing this command. When advertising is disabled using the LE_Set_Extended_Advertising_Enable command (see Section 7.8.57), the packets containing the AUX_SYNC_IND PDUs and the Supplemental are no longer transmitted.

Once enabled, the Controller shall continue advertising with Supplementals until the Host issues an LE_Set_Connectionless_Supplemental_Transmit_Enable command with Supplemental_Enable set to 0x00 (disabling Supplementals) or if the Host issues an LE_Set_Extended_Advertising_Enable command (see Section 7.8.57) with Advertising_Enable set to 0x00 (disabling advertising).

If the Host issues this command before it has issued the LE_Set_Connectionless_Supplemental_Transmit_Parameters command for the advertising set, the Controller shall return the error code Command Disallowed (0x0C).

If the advertising set corresponding to the Advertising_Handle parameter does not exist, the Controller shall return the error code Unknown Advertising Identifier (0x42).

The Host may issue this command when advertising is enabled in the advertising set.

# Command Parameters:

| Advertising_Handle: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 – 0xEF | Identifier for the advertising set in which Supplemental is being enabled or disabled. |  |
| All other values | Reserved for future use |  |

| Supplemental_Enable: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Advertising with Supplemental is disabled (default) |  |
| 0x01 | Advertising with Supplemental is enabled |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Set_Connectionless_Supplemental_Transmit_Enable command succeeded. |  |
| 0x01 – 0xFF | LE_Set_Connectionless_Supplemental_Transmit_Enable command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Set_Connectionless_Supplemental_Transmit_Enable command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+4 LE Read Connectionless I&Q Sampling List Size Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Read_Connectionless_I&Q_Sampling_List_Size | TBD |  | Status, I&Q_Sampling_List_Size |

# Description:

The LE_Read_Connectionless_I&Q_Sampling_List_Size command is used to read the maximum number of I&Q_sampling list entries (the list of advertisers from which the Controller is requested to capture I&Q samples) that can be stored in the Controller.

Note: The number of entries that can be stored is not fixed and the Controller can change it at any time (e.g. because the memory used to store the list can also be used for other purposes).

# Command Parameters:

None.

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Read_Connectionless_I&Q_Sampling_List_Size command succeeded. |  |
| 0x01 – 0xFF | LE_Read_Connectionless_I&Q_Sampling_List_Size command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

| I&Q Sampling List Size: |  | Size: 2 Octets |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x0001 – 0xFFFF | Maximum number of I&Q sampling list entries that can be stored in the Controller. |  |
| 0x0000 | Reserved for future use |  |

# Event(s) Generated (unless masked away):

When the LE_Read_Connectionless_I&Q_Sampling_List_Size command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+5 LE Add Device To Connectionless I&Q Sampling List Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Add_Device_To_Connectionless_I&Q_Sampling_List | TBD | Address_Type, Address, Advertising_SID, Slot_Durations | Status, Current_List_Size |

# Description:

The LE_Add_Device_To_Connectionless_I&Q_Sampling_List command is used to add a device to the list of advertisers from which the Controller is requested to take I&Q samples. If the device is already on the list, the Controller shall return the error code Invalid HCI Command Parameters (0x12).

Note: If the address is not on the White List then advertisements from it will be ignored and therefore I&Q samples will not be captured.

The Advertising_SID parameter specifies the value that must be in the Advertising SID subfield in the ADI field of the advertising PDU that identifies the periodic advertising.

If Advertising_SID is set to 0xFF then the Controller shall accept any value for the Advertising_SID subfield.

If the Slot_Durations parameter is set to 0x01 and the Controller does not support 1 µs switching and sampling, the Controller shall return the error code Unsupported Feature or Parameter value (0x11).

The Slot_Durations parameter is only valid when receiving an AoA Supplemental.

# Command Parameters:

| Address_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Public Device Address |  |
| 0x01 | Random Device Address |  |
| 0x02 | Public Identity Address (corresponds to peer’s Resolvable Private Address) |  |
| 0x03 | Random (static) Identity Address (corresponds to peer’s Resolvable Private Address) |  |
| All other values | Reserved for future use |  |

| Address: | Size: 6 Octets |  |
| --- | --- | --- |
| Value |  | Parameter Description |
| 0xXXXXXXXXXXXX |  | Public Device Address, Random Device Address, Public Identity Address or Random (static) Identity Address of the device whose advertising PDU is being reported. |

| Advertising_SID: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 – 0x0F | Value that must be in the Advertising SID subfield in the ADI field. |  |
| 0xFF | Accept any value in the Advertising_SID subfield |  |
| All other values | Reserved for future use |  |

| Slot_Durations: |  |  | Size: 1 Octet |
| --- | --- | --- | --- |
| Value | Parameter Description |  |  |
| 0x01 | Switching and sampling slots are 1 µs each |  |
| 0x02 | Switching and sampling slots are 2 µs each |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  |  | Size: 1 Octet |  |
| --- | --- | --- | --- | --- |
| Value |  | Parameter Description |  |  |
| 0x00 |  | LE_Add_Device_To_Connectionless_I&Q_Sampling_List command succeeded. |  |  |
| 0x01 – 0xFF |  | LE_Add_Device_To_Connectionless_I&Q_Sampling_List command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |  |

| Current_List_Size: |  |  | Size: 2 Octets |
| --- | --- | --- | --- |
| Value | Parameter Description |  |
| 0x0000 – 0xFFFE | Number of I&Q sampling list entries that can still be stored in the Controller after this command has completed. |  |  |
| 0xFFFF | Current_List_Size not reported. |  |  |

# Event(s) Generated (unless masked away):

When the LE_Add_Device_To_Connectionless_I&Q_Sampling_List command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+6 LE Remove Device From Connectionless I&Q Sampling List Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Remove_Device_From_Connectionless_I&Q_Sampling_List | TBD | Address_Type, Address, Advertising_SID, Slot_Durations | Status, Current_List_Size |

# Description:

The LE_Remove_Device_From_Connectionless_I&Q_Sampling_List command is used to remove a single device from the list of advertisers from which the Controller is requested to take I&Q samples.

If the Controller cannot remove a device from the list because it is not found, the Controller shall return the error code Unknown Advertising Identifier (0x42).

# Command Parameters:

| Address_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Public Device Address |  |
| 0x01 | Random Device Address |  |
| 0x02 | Public Identity Address (corresponds to peer’s Resolvable Private Address) |  |
| 0x03 | Random (static) Identity Address (corresponds to peer’s Resolvable Private Address) |  |
| All other values | Reserved for future use |  |

| Address: | Size: 6 Octets |  |
| --- | --- | --- |
| Value |  | Parameter Description |
| 0xXXXXXXXXXXXX |  | The Address of the device which is to be removed from the list. |

| Advertising_SID: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 – 0x0F | Remove the entry for the advertiser if the Advertising_SID matches this value. |  |
| 0xFF | Remove all entries for the advertiser irrespective of Advertising_SID. |  |
| All other values | Reserved for future use |  |

| Slot_Durations: |  |  | Size: 1 Octet |
| --- | --- | --- | --- |
| Value | Parameter Description |  |  |
| 0x01 | Switching and sampling slots are 1 µs each |  |
| 0x02 | Switching and sampling slots are 2 µs each |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  |  | Size: 1 Octet |  |
| --- | --- | --- | --- | --- |
| Value |  | Parameter Description |  |  |
| 0x00 |  | LE_Remove_Device_From_Connectionless_I&Q_Sampling_List command succeeded. |  |  |
| 0x01 – 0xFF |  | LE_Remove_Device_From_Connectionless_I&Q_Sampling_List command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |  |

| Current_List_Size: |  |  | Size: 2 Octets |
| --- | --- | --- | --- |
| Value | Parameter Description |  |
| 0x0000 – 0xFFFE | Number of I&Q sampling list entries that can still be stored in the Controller after this command has completed. |  |  |
| 0xFFFF | Current_List_Size not reported. |  |  |

# Event(s) Generated (unless masked away):

When the LE_Remove_Device_From_Connectionless_I&Q_Sampling_List command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+7 LE Clear Connectionless I&Q Sampling List Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Clear_Connectionless_I&Q_Sampling_List | TBD |  | Status |

# Description:

The LE_Clear_Connectionless_I&Q_Sampling_List command is used to clear the list of the advertiser addresses stored for I&Q sampling in the Controller.

# Command Parameters:

None.

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Clear_Connectionless_I&Q_Sampling_List command succeeded. |  |
| 0x01 – 0xFF | LE_Clear_Connectionless_I&Q_Sampling_List command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Clear_Connectionless_I&Q_Sampling_List command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+8 LE Set Connectionless I&Q Sampling Enable Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Set_Connectionless_I&Q_Sampling_Enable | TBD | Sampling_Enable, Sampling_Filter_Policy, Address_Type, Address, Advertising_SID, Number_of_Antenna_IDs, Antenna_IDs[i], Slot_Durations | Status |

# Description:

The LE_Set_Connectionless_I&Q_Sampling_Enable command is used to request that the Controller enables or disables capturing I&Q samples from the Supplementals of periodic advertising packets.

If the Sampling_Enable parameter is set to 0x01 (sampling is enabled), the Controller starts attempting to receive AUX_SYNC_IND PDUs.

The Sampling_Filter_Policy parameter is used to determine whether the I&Q sampling list is used. If the I&Q Sampling List is not used, the Address_Type, Address, and Advertising_SID parameters specify the advertisements to report.

The Advertising_SID parameter specifies the value that must be in the Advertising SID subfield in the ADI field of the advertising PDU that identifies the periodic advertising. If Advertising_SID is set to 0xFF then the Controller shall accept any value for the Advertising_SID subfield.

Scanning must also be enabled for I&Q sampling to take place using the LE_Set_Extended_Scan_Enable command (Section 7.8.66). Once the periodic advertising stream(s) has been acquired, samples will continue to be reported even if the Host disables scanning.. The scan parameters are set using the LE_Set_Extended_Scan_Parameters command (Section 7.8.65).

Once the stream has been acquired, the Controller shall continue taking I&Q samples until the Host issues an LE_Set_Connectionless_I&Q_Enable command with Sampling_Enable set to 0x00 (sampling is disabled) or synchronization with the periodic advertising stream is lost.).

The command is also used to set the antenna switching pattern and switching and sampling slot durations to be used while receiving the Supplemental.

If the Slot_Durations parameter is set to 0x01 and the Controller does not support 1 µs switching and sampling, the Controller shall return the error code Unsupported Feature or Parameter value (0x11).

The Slot_Durations parameter is only valid when receiving an AoA Supplemental.

# Command Parameters:

| Sampling_Enable: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Connectionless I&Q sampling is disabled (default) |  |
| 0x01 | Connectionless I&Q sampling is enabled |  |
| All other values | Reserved for future use |  |

| Sampling_Filter_Policy: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Use Address_Type, Address and Advertising_SID to determine which advertisements to report. |  |
| 0x01 | Use the I&Q Sampling List to determine which advertisements to report. |  |
| All other values | Reserved for future use |  |

| Address_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Public Device Address |  |
| 0x01 | Random Device Address |  |
| 0x02 | Public Identity Address (corresponds to peer’s Resolvable Private Address) |  |
| 0x03 | Random (static) Identity Address (corresponds to peer’s Resolvable Private Address) |  |
| All other values | Reserved for future use |  |

| Address: | Size: 6 Octets |  |
| --- | --- | --- |
| Value |  | Parameter Description |
| 0xXXXXXXXXXXXX |  | The Advertiser’s Address |

| Advertising_SID: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 – 0x0F | Value that must be in the Advertising SID subfield in the ADI field. |  |
| 0xFF | No Advertising_SID specified; accept any value for Advertising_SID |  |
| All other values | Reserved for future use |  |

| Number_of_Antenna_IDs: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x01 – 0x4B | The number of Antenna IDs in the sequence |  |

| Antenna_IDs[i]: |  | Size: Number_of_Antenna_IDs * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | List of Antenna IDs in the sequence. The Antenna IDs shall take values from 0 to Number_of_Antenna_IDs-1 inclusive. |  |

Return Parameters:

| Slot_Durations: |  |  | Size: 1 Octet |
| --- | --- | --- | --- |
| Value | Parameter Description |  |  |
| 0x01 | Switching and sampling slots are 1 µs each |  |
| 0x02 | Switching and sampling slots are 2 µs each |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  |  | Size: 1 Octet |  |
| --- | --- | --- | --- | --- |
| Value |  | Parameter Description |  |  |
| 0x00 |  | LE_Set_Connectionless_I&Q_Sampling_Enable command succeeded. |  |  |
| 0x01 – 0xFF |  | LE_Set_Connectionless_I&Q_Sampling_Enable command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |  |

# Event(s) Generated (unless masked away):

When the LE_Set_Connectionless_I&Q_Sampling_Enable command has completed, a Command Complete event shall be generated.

LE Connectionless I&Q Report events are generated by the Controller based on the advertising packets received.

[NEW SECTION] 7.8.XX+9 LE Set Connection Supplemental Receive Parameters Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Set_Connection_Supplemental_Receive_Parameters | TBD | Connection_Handle, Number_of_Antenna_IDs, Antenna_IDs[i], Slot_Durations | Status |

# Description:

The LE_Set_Connection_Supplemental_Receive_Parameters command is used to set the antenna switching pattern and switching and sampling slot durations for receiving Supplementals on the connection identified by the Connection_Handle parameter.

If the Host issues this command when Supplemental requests have been enabled on the connection, the Controller shall return the error code Command Disallowed (0x0C).

If the Slot_Durations parameter is set to 0x01 and the Controller does not support 1 µs switching and sampling, the Controller shall return the error code Unsupported Feature or Parameter value (0x11).

The Slot_Durations parameter is only valid when receiving an AoA Supplemental.

# Command Parameters:

| Connection_Handle: |  | Size: 2 Octets (12 bits meaningful) |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXXXX | Connection_Handle Range 0x0000 – 0x0EFF (0x0F00 – 0x0FFF Reserved for future use) |  |

| Number_of_Antenna_IDs: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x01 – 0x4B | The number of Antenna IDs in the sequence |  |
| All other values | Reserved for future use |  |

| Antenna_IDs[i]: |  | Size: Number_of_Antenna_IDs * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | List of Antenna IDs in the sequence. The Antenna IDs shall take values from 0 to Number_of_Antenna_IDs-1 inclusive. |  |

| Slot_Durations: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x01 | Switching and sampling slots are 1 µs each |  |
| 0x02 | Switching and sampling slots are 2 µs each |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Set_Connection_Supplemental_Receive_Parameters command succeeded. |  |
| 0x01 – 0xFF | LE_Set_Connection_Supplemental_Receive_Parameters command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Set_Connection_Supplemental_Receive_Parameters command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+10 LE Set Connection Supplemental Transmit Parameters Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Set_Connection_Supplemental_Transmit_Parameters | TBD | Connection_Handle, Number_of_Antenna_IDs, Antenna_IDs[i], Slot_Durations | Status |

# Description:

The LE_Set_Connection_Supplemental_Transmit_Parameters command is used to set the antenna switching pattern, switching and sampling slot durations used for transmitting Supplementals on the connection identified by the Connection_Handle parameter.

If the Host issues this command when Supplemental responses have been enabled on the connection, the Controller shall return the error code Command Disallowed (0x0C).

If the Slot_Durations parameter is set to 0x01 and the Controller does not support 1 µs switching and sampling, the Controller shall return the error code Unsupported Feature or Parameter value (0x11).

# Command Parameters:

| Connection_Handle: |  | Size: 2 Octets (12 bits meaningful) |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXXXX | Connection_Handle Range 0x0000 – 0x0EFF (0x0F00 – 0x0FFF Reserved for future use) |  |

| Number_of_Antenna_IDs: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x01 – 0x4B | The number of Antenna IDs in the sequence. |  |
| All other values | Reserved for future use |  |

| Antenna_IDs[i]: |  | Size: Number_of_Antenna_IDs * 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXX | List of Antenna IDs in the sequence. The Antenna IDs shall take values from 0 to Number_of_Antenna_IDs-1 inclusive. |  |

| Slot_Durations: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | No switching and sampling |  |
| 0x01 | Switching and sampling slots are 1 µs each |  |
| 0x02 | Switching and sampling slots are 2 µs each |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Set_Connection_Supplemental_Transmit_Parameters command succeeded. |  |
| 0x01 – 0xFF | LE_Set_Connection_Supplemental_Transmit_Parameters command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Set_Connection_Supplemental_Transmit_Parameters command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+11 LE Connection Supplemental Request Enable Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Connection_Supplemental_Request_Enable | TBD | Connection_Handle, Enable, Supplemental_Request_Interval, Requested_Supplemental_Length, Requested_Supplemental_Type | Status |

# Description:

The LE_Connection_Supplemental_Request_Enable command is used to request the Controller to start or stop sending LL_SUPPLEMENTAL_REQ PDU(s) on a connection identified by the Connection_Handle parameter.

The Supplemental_Request_Interval parameter defines whether the LL_SUPPLEMENTAL_REQ PDU is sent only once or periodically. In the case of periodic operation, the LL_SUPPLEMENTAL_REQ PDU is sent every Supplemental_Request_Interval. However, the Controller may delay sending the LL_SUPPLEMENTAL_REQ PDU beyond the requested interval, e.g., in order to prioritize other activities. The Supplemental_Request_Interval shall be greater than or equal to (connSlaveLatency + 1).

The Requested_Supplemental_Length parameter indicates the minimum length of the Supplemental and the Requested_Supplemental_Type parameter indicates the type of Supplemental that the Controller shall request from the remote device.

If the Host issues this command before issuing the LE_Set_Connection_Supplemental_Receive_Parameters command at least once on the connection, the Controller shall return the error code Command Disallowed (0x0C).

If the Host issues this command when the receiver PHY for the connection is the LE Coded PHY, the Controller shall return the error code Command Disallowed (0x0C).

If the receiver PHY for the connection changes to the LE Coded PHY then the Controller shall automatically disable PHY Supplemental requests.

# Command Parameters:

| Connection_Handle: |  | Size: 2 Octets (12 bits meaningful) |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXXXX | Connection_Handle Range 0x0000 – 0x0EFF (0x0F00 – 0x0FFF Reserved for future use) |  |

| Enable: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | Disable Supplemental Request for the connection (default) |  |
| 0x01 | Enable Supplemental Request for the connection |  |
| All other values | Reserved for future use |  |

| Supplemental_Request_Interval: |  | Size: 2 Octets |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x0000 | Send LL_SUPPLEMENTAL_REQ once, at the earliest practical opportunity |  |
| 0x0001 – 0xFFFF | Requested interval for sending LL_SUPPLEMENTAL_REQ PDUs in number of connection events. |  |

| Requested_Supplemental_Length: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x02 – 0x14 | Minimum length of the Supplemental being requested in 8 µs units |  |
| All other values | Reserved for future use |  |

| Requested_Supplemental_Type: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | AoA Supplemental |  |
| 0x01 | AoD Supplemental with 1 µs slots |  |
| 0x02 | AoD Supplemental with 2 µs slots |  |
| All other values | Reserved for future use |  |

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Connection_Supplemental_Request_Enable command succeeded. |  |
| 0x01 – 0xFF | LE_Connection_Supplemental_Request_Enable command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Connection_Supplemental_Request_Enable command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+12 LE Connection Supplemental Response Enable Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Connection_Supplemental_Response_Enable | TBD | Connection_Handle, Enable | Status |

# Description:

The LE_Connection_Supplemental_Response_Enable command is used to request the Controller to respond to LL_SUPPLEMENTAL_REQ PDUs with LL_SUPPLEMENTAL_RSP PDUs on the specified connection.

If the Host issues this command before issuing the LE Set Connection Supplemental Transmit Parameters command at least once on the connection, the Controller shall return the error code Command Disallowed (0x0C).

If the Host issues this command when the transmitter PHY for the connection is the LE Coded PHY, the Controller shall return the error code Command Disallowed (0x0C).

If the transmitter PHY for the connection changes to the LE Coded PHY then the Controller shall automatically disable PHY Supplemental responses.

# Command Parameters:

| Connection_Handle: |  | Size: 2 Octets (12 bits meaningful) |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0xXXXX | Connection_Handle Range 0x0000 – 0x0EFF (0x0F00 – 0x0FFF Reserved for future use) |  |

| Enable: |  | Size: 1 Octet |
| --- | --- | --- |
| Bit number | Meaning |  |
| 0 | Enable AoA Supplemental Response |  |
| 1 | Enable AoD Supplemental with 1 µs slots Response |  |
| 2 | Enable AoD Supplemental with 2 µs slots Response |  |
| All other bits | Reserved for future use |  |

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Connection_Supplemental_Response_Enable command succeeded. |  |
| 0x01 – 0xFF | LE_Connection_Supplemental_Response_Enable command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

# Event(s) Generated (unless masked away):

When the LE_Connection_Supplemental_Response_Enable command has completed, a Command Complete event shall be generated.

[NEW SECTION] 7.8.XX+13 LE Read Maximum Supplemental Length Command

| Command | OCF | Command Parameters | Return Parameters |
| --- | --- | --- | --- |
| HCI_LE_Read_Maximum_Supplemental_Length | TBD |  | Status, MaximumSupplementalLength |

# Description:

The LE_Read_Maximum_Supplemental_Length command allows the Host to read the maximum length of Supplemental that the Controller can transmit.

# Command Parameters:

None.

# Return Parameters:

| Status: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x00 | LE_Read_Maximum_Supplemental_Length command succeeded. |  |
| 0x01 – 0xFF | LE_Read_Maximum_Supplemental_Length command failed. See [Vol 2] Part D, Error Codes for a list of error codes and descriptions. |  |

| MaximumSupplementalLength: |  | Size: 1 Octet |
| --- | --- | --- |
| Value | Parameter Description |  |
| 0x02 – 0x14 | Maximum length of Supplemental supported in 8 µs units |  |
| All other values | Reserved for future use |  |

# Event(s) Generated (unless masked away):

When the LE_Read_Maximum_Supplemental_Length command has completed, a Command Complete event shall be generated.

Volume 3, Part C (Generic Access Profile) Changes

Note to the reviewers: this CR makes no changes to GAP. Justification:

- Connectionless AoD only adds a Supplemental to the AUX_SYNC_IND. Thus, no GAP changes beyond those made under the Advertising Extensions [2] feature are required.

- Connection-oriented AoA does not warrant new GAP roles other than the existing Central and Peripheral roles.

- I&Q samples generated by the Controller are not processed by GAP.

Volume 6, Part A (Physical Layer) Changes

[NEW SECTION] 4.X ANTENNA SWITCHING

A device may support an antenna array consisting of two or more antennae that are controlled by a switch. The device switches between the antennae either while receiving the Supplemental part of a packet (Angle of Arrival method) or while transmitting the Supplemental part of a packet (Angle of Departure method). The switching takes place during time periods called switch slots. The first 4 µs of the Supplemental are termed the guard period and the next 8 µs are termed the reference period. The Link Layer captures I&Q samples during the reference period and during time periods called sample slots.

When a Controller that supports two or more antennae transmits a packet containing an AoD Supplemental or receives a packet containing an AoA Supplemental, it shall switch the antennae according to the switching sequence configured by its Host.

The first antenna in the sequence shall be used during the guard period and the reference period (see [Vol 6] Part B, Section 2.X.1 Supplemental format). The second antenna in the sequence shall be used during the first sample slot, the third antenna during the second sample slot, and so on. The antenna in use shall only be changed during the switch slots.

If the sequence specified by the Host is exhausted before the last sample slot, it shall be restarted from the beginning (this can happen more than once); that is, the first antenna in the sequence is used in the sample slot following that used for the last antenna in the sequence. If the sequence has not been completely used by the end of the Supplemental, any remaining terms shall be ignored.

Volume 6, Part B (Link Layer) Changes

[UPDATED SECTION] 2.1 PACKET FORMAT FOR LE UNCODED PHYS

[Note: The Advertising Extension CR [2] and the Long Range feature [1] also modify this section. Those changes have been considered accepted here]

The following packet format is defined for the LE Uncoded PHYs (LE 1M and LE 2M) and is used for both advertising channel packets and data channel packets.

This packet format is shown in Figure 2.1. Each packet consists of four mandatory fields and one optional field. The mandatory fields are Preamble, Access Address, PDU, and CRC.  The optional field is Supplemental.

LSB												MSB

| Preamble (1 or 2 octets) | Access Address (4 octets) | PDU (2-2578 octets) | CRC (3 octets) | Supplemental (16 to 160 microseconds) |
| --- | --- | --- | --- | --- |

Figure 2.1: Link Layer packet format

The preamble is 1 octet when transmitting or receiving on the LE 1M PHY and 2 octets when transmitting or receiving on the LE 2M PHY. The Access Address is 4 octets. The PDU range is from 2 to 257258 octets. The CRC is 3 octets.

The Preamble is transmitted first, followed by the Access Address, followed by the PDU, followed by the CRC, and Supplemental (if present) in that order. The entire packet is transmitted at the same symbol rate (either 1Ms/s or 2Ms/s modulation).

# Packets (not including the Supplemental) take between 44 and 2120 µs to transmit.

When the Supplemental is present, the Supplemental duration is between 16 and 160 microseconds, as shown in Figure 2.1.

[UPDATED SECTION] 2.1.4 CRC

At the end of every Link Layer packet there isThe PDU is followed by a 24-bit CRC. It shall be calculated over the PDU. The CRC polynomial is defined in Section 3.1.1.

[NEW SECTION] 2.1.X Supplemental

The CRC is followed by an optional Supplemental which consists of a constantly modulated series of unwhitened 1s. The Supplemental is not included in CRC or MIC calculations.

[NEW SECTION] 2.3.X.3 SuppInfo

[Note: Section 2.3.X (Common Advertising Payload Format) was introduced by the LE Advertising Extension CR [2]]

The presence of the SuppInfo field indicates that the packet includes a Supplemental. The SuppInfo field is defined in Section 2.X.2.

[UPDATED SECTION] 2.4 DATA CHANNEL PDU

The Data Channel PDU has a 16 or 24 bit header, a variable size payload, and may include a Message Integrity Check (MIC) field.

# The Data Channel PDU is as shown in Figure 2.13.

[Modify Figure 2.13 as shown below]

# Figure 2.13: Data Channel PDU

# The Header field of the Data Channel PDU is as shown in Figure 2.14.

[Modify Figure 2.14 as shown below]

# Figure 2.14: Data channel PDU header

# The 16 or 24 bit Header field consists of 56 or 7 fields that are specified in Table 2.3.

The MIC field shall not be included in an un-encrypted Link Layer connection,

or in an encrypted Link Layer connection with a data channel PDU with a zero

length Payload.

# The MIC field shall be included in an encrypted Link Layer connection, with a

data channel PDU with a non-zero length Payload and shall be calculated as

specified in [Vol. 6] Part E, Section 1.

# The payload format depends on the LLID field of the Header. If the LLID field is

# 01b or 10b, the Data Channel PDU Payload field contains an LL Data PDU as

defined in Section 2.4.1. If the LLID field is 11b then the Data Channel PDU

# Payload field contains an LL Control PDU as defined in Section 2.4.2. An LLID

field of 00b is reserved.

# The NESN bit of the Header is defined in Section 4.5.9.

# The SN bit of the Header is defined in Section 4.5.9.

# The MD bit of the Header is defined in Section 4.5.6.

The SuppInfo Present (SP) field of the Header indicates whether the data channel PDU Header has a SuppInfo field and therefore whether the data channel packet has a Supplemental. If the SP field is 0, then no SuppInfo field is present in the data channel PDU Header and there is no Supplemental in the data channel packet. If the SP field is 1, then the SuppInfo field in the data channel PDU Header is present and the data channel packet includes a Supplemental.

The Length field of the Header indicates the length of the Payload and MIC if included. The length field has the range of 0 to 255 octets. The Payload field shall be less than or equal to 251 octets in length. The MIC is 4 octets in length.

The SuppInfo field of the Header is present if indicated in the SP field. The SuppInfo field is defined in Section 2.X.2. The SuppInfo field can be present in the header of any Data Channel PDU. However, the Link Layer shall not transmit a Data Channel PDU containing a SuppInfo field unless the peer device, in the same connection, has already transmitted any of:

- a Data Channel PDU containing a SuppInfo field;

- an LL_FEATURE_REQ, LL_FEATURE_RSP, or LL_SLAVE_FEATURE_REQ PDU with the "Connection Supplemental Request" feature bit set; or

- an LL_SUPPLEMENTAL_REQ or LL_SUPPLEMENTAL_RSP PDU.

In this version of the Core Specification, the only way to request a remote device to send a packet containing the SuppInfo field is via the Supplemental Request control procedure.

| Field Name | Description |
| --- | --- |
| LLID | The LLID indicates whether the packet is an LL Data PDU or an LL Control PDU. 00b = Reserved 01b = LL Data PDU: Continuation fragment of an L2CAP message, or an Empty PDU. 10b = LL Data PDU: Start of an L2CAP message or a complete L2CAP message with no fragmentation. 11b = LL Control PDU |
| NESN | Next Expected Sequence Number |
| SN | Sequence Number |
| MD | More Data |
| SP | SuppInfo Present |
| Length | The Length field indicates the size, in octets, of the Payload and MIC, if included. |
| SuppInfo | The SuppInfo field indicates the type and length of the Supplemental. |

Table 2.3: Data channel PDU Header field

[UPDATED SECTION] 2.4.2 LL Control PDU

| Opcode | Control PDU Name |
| --- | --- |
| 0x00 | LL_CONNECTION_UPDATE_REQ |
| 0x01 | LL_CHANNEL_MAP_REQ |
| 0x02 | LL_TERMINATE_IND |
| 0x03 | LL_ENC_REQ |
| 0x04 | LL_ENC_RSP |
| 0x05 | LL_START_ENC_REQ |
| 0x06 | LL_START_ENC_RSP |
| 0x07 | LL_UNKNOWN_RSP |
| 0x08 | LL_FEATURE_REQ |
| 0x09 | LL_FEATURE_RSP |
| 0x0A | LL_PAUSE_ENC_REQ |
| 0x0B | LL_PAUSE_ENC_RSP |
| 0x0C | LL_VERSION_IND |
| 0x0D | LL_REJECT_IND |
| 0x0E | LL_SLAVE_FEATURE_REQ |
| 0x0F | LL_CONNECTION_PARAM_REQ |
| 0x10 | LL_CONNECTION_PARAM_RSP |
| 0x11 | LL_REJECT_ EXT_IND |
| 0x12 | LL_PING_REQ |
| 0x13 | LL_PING_RSP |
| 0x14 | LL_LENGTH_REQ |
| 0x15 | LL_LENGTH_RSP |
| 0x16 | LL_PHY_REQ |
| 0x17 | LL_PHY_RSP |
| 0x18 | LL_PHY_UPDATE_IND |
| TBD | LL_SUPPLEMENTAL_REQ |
| TBD+1 | LL_SUPPLEMENTAL_RSP |
| All other values | Reserved for Future Use |

Table 2.4: LL Control PDU Opcodes

[NEW SECTION] 2.4.2.X LL_SUPPLEMENTAL_REQ

The format of the CtrData field is shown in Figure 2.X.

| CtrData |  |
| --- | --- |
| MinSupLenReq (6 bits) | SupTypeReq (2 bits) |

Figure 2.X: CtrData field of the LL_SUPPLEMENTAL_REQ PDU

The LL_SUPPLEMENTAL_REQ PDU CtrData consists of two fields:

- MinSupLenReq shall contain the minimum length of the Supplemental (see Section 2.4.2.X+1.1) requested from the remote device, in 8 µs units. The value of the field shall be between 2 and 20 inclusive. All other values are reserved for future use.

- SupTypeReq shall contain the type of the Supplemental requested from the remote device as specified in Table 2.Y.

| Value | Meaning |
| --- | --- |
| 0 | AoA Supplemental (see section 2.X.1) |
| 1 | AoD Supplemental with 1 µs slots (see section 2.X.1) |
| 2 | AoD Supplemental with 2 µs slots (see section 2.X.1) |
| 3 | Reserved for future use |

Table 2.Y: SupTypeReq field bit meanings

[NEW SECTION] 2.4.2.X+1 LL_SUPPLEMENTAL_RSP

# The LL_SUPPLEMENTAL_RSP PDU does not have a CtrData field.

[UPDATED SECTION] 2.5 PACKET FORMAT FOR THE LE CODED PHY

[Add at the end of the section]

Note: There is no Supplemental on the LE Coded PHY.

[NEW SECTION] 2.X Supplemental and I&Q Sampling

[NEW SECTION] 2.X.1 Supplemental structure and types

The Supplemental has a variable length. The contents are always a constantly modulated series of 1s and no whitening shall be applied to them.

The first 4 µs of the Supplemental are termed the guard period and the next 8 µs are termed the reference period. After the reference period the Supplemental consists of a sequence of alternating switch slots and sample slots, each either 1 µs or 2 µs long as specified by the Host. The 2-µs-long switch and sample slots are mandatory to support, the 1-µs-long switch and sample slots are optional to support. The structure of the Supplemental is shown in Figure X.X+1.

# The Supplemental can be of two types: AoA or AoD.

# Figure X.X+1: Supplemental structure

The Supplemental can be of two types: AoA or AoD.

[NEW SECTION] 2.X.2 SuppInfo field

When present, the SuppInfo field is one octet with the format shown in figure 2.X.

| SuppInfo |  |  |
| --- | --- | --- |
| SupplementalTime (5 bits) | RFU (1 bit) | SupplementalType (2 bits) |

Figure 2.X: SuppInfo field

# The presence of the SuppInfo field indicates that the packet includes a Supplemental.

The SupplementalTime field defines the length of the Supplemental in 8 µs units. The value of the SupplementalTime field shall be between 2 and 20 inclusive; all other values are reserved for future use.

The SupplementalType field defines the type of the Supplemental and the duration of the switching slots. The value of the field is specified in table Y.Y and the various possible formats are specified in Section 2.X.1.

| SupplementalType value | Description |
| --- | --- |
| 0 | AoA Supplemental |
| 1 | AoD Supplemental with 1 µs slots |
| 2 | AoD Supplemental with 2 µs slots |
| 3 | Reserved for future use |

Table Y.Y: SupplementalType field encoding

[NEW SECTION] 2.X.3 Transmitting Supplementals

When transmitting a packet that contains an AoA Supplemental, the transmitter shall not switch antennae. When transmitting a packet that contains an AoD Supplemental, the transmitter shall perform antenna switching at the rate and according to the switching sequence configured by the Host (see [Vol 6] Part A, Section 4.X).

[NEW SECTION] 2.X.4 I&Q Sampling

When receiving a packet that contains an AoD Supplemental, the receiver does not need to switch antennae. When receiving a packet that contains an AoA Supplemental, the receiver shall perform antenna switching at the rate and according to the switching sequence configured by the Host (see [Vol 6] Part A, Section 4.X). In both cases the receiver shall take an I&Q sample each microsecond during the reference period and an I&Q sample each sample slot (thus there will be 8 reference I&Q samples, 1 to 37 I&Q samples with 2 µs slots, and 2 to 74 I&Q samples with 1 µs slots, meaning 9 to 82 samples in total). The Controller shall report the I&Q samples to the Host.

Note: In order to obtain good quality data for angle estimation, I&Q samples should be taken at the same point within each I&Q Sampling Window; this starts 0.125 µs after the beginning and ends 0.125 µs before the end of each microsecond period  (see Figure X.X+2). If 2 µs sample slots are used, the sampling should be done during the latter microsecond.

# Figure X.X+2: I&Q Sampling Window for 1 µs sample slots.

# Figure X.X+3: I&Q Sampling Window for 2 µs sample slots.

[UPDATED SECTION] 3.2 DATA WHITENING

Data whitening is used to avoid long sequences of zeros or ones, e.g. 0000000b or 1111111b, in the data bit stream. Whitening shall be applied on the PDU and CRC fields of all Link Layer packets and is performed after the CRC generation in the transmitter. No other parts of the packets are whitened. De-whitening is performed before the CRC checking in the receiver (see Figure 3.1).

[UPDATED SECTION] 4.6 FEATURE SUPPORT

| Bit position | Link Layer Feature | Valid from Controller to Host | Valid from Controller to Controller |
| --- | --- | --- | --- |
| …. |  |  |  |
| 7 | Extended Scanner Filter Policies | Y | Y |
| 8 | LE 2M PHY | Y | Y |
| 9 | Stable Modulation Index – Transmitter | Y | Y |
| 10 | Stable Modulation Index – Receiver | Y | Y |
| 11 | LE Coded PHY | Y | Y |
| TBD1 | Connection Supplemental Request | Y | Y |
| TBD2 | Connection Supplemental Response | Y | Y |
| TBD3 | Connectionless Supplemental Transmitter | Y | N |
| TBD4 | Connectionless Supplemental Receiver | Y | N |
| TBD5 | Antenna switching During Supplemental Transmission (AoD) | Y | N |
| TBD6 | Antenna Switching During Supplemental Reception (AoA) | Y | N |
| All other values | Reserved for Future Use |  |  |

Table 4.4: FeatureSet field’s bit mapping to Controller features

[NEW SECTION] 4.6.X Connection Supplemental Request

A Controller that supports Connection Supplemental Request shall support the following sections of this document.

- LL_REJECT_EXT_IND (2.4.2.18)

- LL_SUPPLEMENTAL_REQ (2.4.2.X)

- LL_SUPPLEMENTAL_RSP (2.4.2.X+1)

- I&Q Sampling (2.X.4)

- Supplemental Request Procedure (5.1.X)

[NEW SECTION] 4.6.X+1 Connection Supplemental Response

A Controller that supports AoA Response shall support the following sections of this document.

- LL_REJECT_EXT_IND (2.4.2.18)

- LL_SUPPLEMENTAL_REQ (2.4.2.X)

- LL_SUPPLEMENTAL_RSP (2.4.2.X+1)

- Transmitting Supplementals (2.X.3)

- Supplemental Request Procedure (5.1.X)

[NEW SECTION] 4.6.X+2 Connectionless Supplemental Transmitter

A Controller that supports Connectionless Supplemental Transmitter shall support the following sections of this document.

- Transmitting Supplementals (2.X.3)

[NEW SECTION] 4.6.X+3 Connectionless Supplemental Receiver

A Controller that supports Connectionless Supplemental Receiver shall support the following sections of this document.

- I&Q Sampling (2.X.4)

[NEW SECTION] 4.6.X+4 Antenna Switching During Supplemental Transmission (AoD)

A Controller that supports Antenna Switching During Supplemental Transmission (AoD) shall support the following sections of this document.

- Transmitting Supplementals (2.X.3)

- Antenna Switching ([Vol 6] Part A, Section 4.X)

[NEW SECTION] 4.6.X+5 Antenna Switching During Supplemental Reception (AoA)

A Controller that supports Antenna Switching During Supplemental Reception (AoA) shall support the following sections of this document.

- I&Q Sampling (2.X.4)

- Antenna Switching ([Vol 6] Part A, Section 4.X)

[NEW SECTION] 5.1.X Supplemental Request Procedure

The Supplemental Request procedure, when supported, is used to request the remote Link Layer to send a LL_SUPPLEMENTAL_RSP PDU with a Supplemental (see Section 2.X.3).

Either the master or the slave Link Layer may initiate this procedure at any time after entering the Connection State by sending an LL_SUPPLEMENTAL_REQ PDU.

If:

- Supplemental responses are enabled;

- the current transmitter PHY is one that supports Supplementals;

- the responding device  is currently configured to respond with the type and length of Supplemental requested in the LL_SUPPLEMENTAL_REQ PDU; and

- a packet containing an LL_SUPPLEMENTAL_RSP PDU and Supplemental of the requested length would not exceed connEffectiveMaxTxTime for the connection (see Section 4.5.10);

then the remote Link Layer shall respond with an LL_SUPPLEMENTAL_RSP PDU that includes a Supplemental of the requested type and whose length is greater than or equal to that requested. Otherwise the remote Link Layer shall respond with an LL_REJECT_EXT_IND PDU.

I&Q sampling (see [Vol 6] Part B, Section 2.X.4) shall be performed while receiving the Supplemental of the LL_SUPPLEMENTAL_RSP PDU and the sampling results shall be reported to the Host.

If the Link Layer of the master or slave sends the LL_SUPPLEMENTAL_REQ PDU to a device that does not understand that PDU, then the device should expect an LL_UNKNOWN_RSP PDU in response.

The procedure is completed when either an LL_SUPPLEMENTAL_RSP PDU has been sent or received or an LL_UNKNOWN_RSP or LL_REJECT_EXT_IND PDU is sent or received with the UnknownType or RejectOpcode (as appropriate) set to LL_SUPPLEMENTAL_REQ.

Volume 6, Part D (Message Sequence Charts) Changes

[NEW SECTION] 2.X Connectionless I&Q Sampling List

The Connectionless I&Q Sampling List may be cleared and a device added as required, before the list is made use of (see Figure 2.X).

# Figure 2.X Connectionless I&Q Sampling List

[NEW SECTION] 3.X Connectionless Supplemental transmission

A device may send periodic advertising packets containing a Supplemental (see Figure 3.X).

# Figure 3.X Connectionless Supplemental Transmission

[NEW SECTION] 4.X Connectionless Supplemental reception

A device may receive periodic advertising packets containing a Supplemental and send I&Q samples to the Host (see Figure 4.X).

# Figure 4.X Connectionless Supplemental Reception

[NEW SECTION] 6.X Supplemental Request

The master or slave of the connection may request the remote device to send an LL_SUPPLEMENTAL_RSP PDU with a Supplemental (see Figure 6.X to Figure 6.X+5).

Figure 6.X Master or Slave-Initiated Supplemental Request Procedure – Remote Device Responds Successfully

Figure 6.X+1 Master or Slave-Initiated Supplemental Request Procedure – Remote Device Does Not Support the Feature

Figure 6.X+2 Master or Slave-Initiated Supplemental Request Procedure – Remote Device Rejects

Volume 6, Part F (Direct Test Mode) Changes

[UPDATED SECTION] 3.3.2 Commands

[Modify the Reset Command table as follows]

| Reset Command: |  |  |  | Size: 12 Bits |  |
| --- | --- | --- | --- | --- | --- |
| Control (6bits) | Parameter (6 bits) | Description |
| 0x00 |  | 0x00 |  | RESET; the upper 2 bits of the data length for any Transmitter or Receiver commands following are set to 00, the PHY is set to 1 Ms/s, and the receiver assumes the transmitter has a standard modulation index, no Supplemental is present, and no antenna array is specified. |  |
|  |  | 0x01 – 0x3F |  | Reserved for future use |  |
| 0x05 |  | 0x00 |  | Read supportedMaxTxOctets (see [Vol 6] Part B, Section 4.5.10) |  |
|  |  | 0x01 |  | Read supportedMaxTxTime (see [Vol 6] Part B, Section 4.5.10) |  |
|  |  | 0x02 |  | Read supportedMaxRxOctets (see [Vol 6] Part B, Section 4.5.10) |  |
|  |  | 0x03 |  | Read supportedMaxTxTime |  |
|  |  | 0x04 |  | Read maximum length of Supplemental supported |  |
|  |  | Any other value |  | Reserved for future use |  |
| 0x06 |  | 0x00 |  | No Supplemental |  |
|  |  | 0x02 – 0x14 |  | Length of the Supplemental in 8 µs units |  |
|  |  | Any other value |  | Reserved for future use |  |
| 0x07 |  | 0x00 |  | Tx: No switching during Supplemental Rx: No Supplemental |  |
|  |  | 0x01 |  | Tx: Switch Supplemental with 1 µs slots. Rx: Sample Supplemental with 1 µs slots. |  |
|  |  | 0x02 |  | Tx: Switch Supplemental with 2 µs slots. Rx: Sample Supplemental with 2 µs slots. |  |
|  |  | Any other value |  | Reserved for future use |  |
| 0x08 |  | 0x00 |  | Antenna array not specified |  |
|  |  | 0x01 – 0x3F |  | Number of antennae in the antenna array |  |
|  |  | Any other value |  | Reserved for future use |  |
| 0x09 |  | 0x00 |  | Antenna switching pattern A: 1, 2, 3, …, n, 1, 2, 3, …, n, … (where n is the number of antennae in the antenna array) |  |
|  |  | 0x01 |  | Antenna switching pattern B: 1, 2, 3, …, n, n-1, n-2, …, 1, … (where n is the number of antennae in the antenna array) |  |
|  |  | Any other value |  | Reserved for future use |  |

If Control 6 is used with a non-zero parameter, Control 7 shall also be used before starting the test. If an AoD Supplemental is selected when transmitting or an AoA Supplemental when receiving, Control 8 shall also be used with a non-zero parameter before starting the test.

If Control 8 is used with a non-zero parameter, Control 9 shall also be used before starting the test.

In the receiver test, Control 6 indicates the expected length of the Supplemental. If the length of the Supplemental in a received LE Test Packet does not match that length, then the DUT shall discard that packet.

[UPDATED SECTION] 3.4.1 LE_Test_Status_Event

[Modify the Reset event bitsCommand table as follows]

| RESET event bits 1 to 14: |  |  |  | Size: 14 Bits |  |
| --- | --- | --- | --- | --- | --- |
| RESET command | Value bits 1 to 14 | Parameter Description |
| 0x04 |  | Bit 4 |  | 0 – Supplemental not supported 1 – Supplemental supported |  |
|  |  | Bit 5 |  | 0 – Antenna switching not supported 1 – Antenna switching supported |  |
|  |  | Bit 6 |  | 0 – 1 µs switching and sampling slots not supported 1 – 1 µs switching and sampling slots supported |  |
|  |  | Bit 47 to 14 |  | Reserved for future use |  |
| 0x05 |  | Bit 1 to 14 |  | One of the following values (depending on the parameter in the original query): - Mmaximum transmit of or receive time divided by 2 , in microseconds, that the local Controller supports for transmission of a single Link Layer Data Channel PDU, divided by 2. or - Mmaximum number of payload octets (depending on the parameter in the original query) that the local Controller supports for transmission of a single Link Layer Data Channel PDU. - Maximum length of the Supplemental that the local Controller supports for transmission in a Link Layer packet, in 8 µs units. Range: - 0x00A4-0x2148 for times or - 0x001B-0x00FF for number of octets (all other values reserved for future use). - 0x02 – 0x14 for the maximum Supplemental length All values outside the range are reserved for future use. |  |

[UPDATED SECTION] 4.1 LE Test Packet Format

[Modify text below. Note that this text is based on the changes made by the Long Range CR i.e. all changes made by the Long Range CR have been ‘accepted’]

The LE Test packet format for LE Uncoded PHYs shall be as shown in Figure 4.1. The LE Test packet format for LE Coded PHY shall be as shown in Figure 4.X. LE test packets are required for LE RF PHY conformance testing using Direct Test Mode.

# Depending on the test, the packet payload content may vary.

For the LE Uncoded PHYs, the LE test packet consists of the following fields: preamble (8 bits when operating at 1 Ms/s modulation or 16 bits when operating at 2 Ms/s modulation), synchronization word (32 bits), PDU header (8 bits), PDU length (8 bits), payload (296-20402048 bits), and CRC (24 bits) and an optional Supplemental (16 – 160 bits when operating at 1 Ms/s modulation or 32-320 bits when operating at 2 Ms/s modulation). LE test packets do not incorporate a PDU address field.

[Modify Figure 4.1 as shown below]

# Figure 4.1: LE Test packet format for the LE Uncoded PHYs

[UPDATED SECTION] 4.1.4 LE Test Packet PDU

The LE test packet PDU consists of an 8-bit header, an 8-bit length field, an optional 8-bit SuppInfo field, and a variable size payload. Its structure is as shown in Figure 4.3.

[Modify Figure 4.3 as shown below. Note that the existing blue color shading should be kept as is. SuppInfo should be a different shade of blue. The PDU payload shape should be kept as is]

# Figure 4.3: LE Test packet PDU structure

[Modify Figure 4.4 as shown below. Note that the existing blue color shading should be kept as is. SuppInfo should be a different shade of blue]

# Figure 4.4: LE Test packet header and length field structure

The first four bits of the PDU header field indicate the payload content type as defined in Table 4.1. The SuppInfo (SP) field of the PDU header indicates whether the SuppInfo field is present and therefore whether the test packet has a Supplemental. If the SP field is 0, then no SuppInfo field is present and there is no Supplemental in the test packet. If the SP field is 1, then the SuppInfo field is present and the test packet includes a Supplemental. The SuppInfo field is defined in [Vol 6] Part B, Section 2.X.2. The length field expresses the payload field length in bytes. The RFU bits shall be set to zero and ignored upon reception.

Note: On the LE Coded PHY, this section defines the PDU contents before coding.

[NEW SECTION] 4.1.X Supplemental

[Add the following paragraph at the end]

The Supplemental is an optional field which consists of a constantly modulated series of unwhitened 1s. It is 16-160 bits when operating at 1 Ms/s modulation or 32-320 bits when operating at 2 Ms/s modulation. The Supplemental is not included in CRC or MIC calculations. The Supplemental shall only be present on the LE Uncoded PHYs.

- References

# Bluetooth Specification, Draft Shanghai d0.9r05

LE Advertising Extension. CR r16. Bluetooth SIG. April 2016.

Connectionless AoD. FIPD r25. Bluetooth SIG. November 2015.

Connection Oriented AoA. FIPD r08. Bluetooth SIG. November 2015.